# An integrated technique for rapid gas permeability measurement of tight rock media

Tao Zhang1, Qinhong Hu1,2*, Behzad Ghanbarian3, Derek Elsworth4, Zhiming Lu5

1 Department of Earth and Environment Sciences, University of Texas at Arlington, Arlington, TX 76019, United States

2National Key Laboratory of Deep Oil and Gas, China University of Petroleum (East China), Qingdao 266580, P.R. China

3 Porous Media Research Lab, Department of Geology, Kansas State University, Manhattan, KS 66506, United States
4 Department of Energy and Mineral Engineering, G3 Centre and Energy Institute, The Pennsylvania State University, University Park, PA 16802, United States

5 The Earth and Environmental Sciences Division, Los Alamos National Laboratory, Los Alamos, NM 87544, United States

Prepared for

Hydrology and Earth System Sciences

June 26, 2023

* Corresponding author: huqinhong@upc.edu.cn

Abstract: Nano-darcy level permeability measurements of porous media, such as nano-porous mudrocks, are frequently conducted with gas invasion methods into granular-sized samples with short diffusion lengths and thereby reduced experimental duration; however, these methods lack rigorous solutions and standardized experimental procedures. For the first time, we resolve this by providing an integrated technique (termed as gas permeability technique) with coupled theoretical development, experimental procedures, and data interpretation workflow. Three exact mathematical solutions for transient and slightly compressible spherical flow, along with their asymptotic solutions, are developed for early- and late-time responses. Critically, one late-time solution is for an ultra-small gas-invadable volume, important for a wide range of practical usages. Developed as applicable to different sample characteristics (permeability, porosity, and mass) in relation to the storage capacity of experimental systems, these three solutions are evaluated from essential considerations of error difference between exact and approximate solutions, optimal experimental conditions, and experimental demonstration of mudrocks and molecular-sieve samples. Moreover, a practical workflow of solution selection and data reduction to determine permeability is presented by considering samples with different permeability and porosity under various granular sizes. Overall, this work establishes a rigorous, theory-based, rapid, and versatile gas permeability measurement technique for tight media at sub-nano darcy levels.

Keywords: permeability; granular samples; pulse-decay; mathematical solutions; experimental methods.

Highlights:

- An integrated (both theory and experiments) gas permeability technique (GPT) is presented.

- Exact and approximate solutions for three cases are developed with error discussion.

- Conditions of each mathematical solution are highlighted for critical parameters.

- Essential experimental methodologies and data processing procedures are provided and evaluated.

# 1. Introduction

Shales, crystalline, and salt rocks with low permeabilities (e.g., <10-17 m2 or 10 micro-darcies μD) are critical components to numerous subsurface studies. Notable examples are the remediation of contaminated sites(Neuzil, 1986; Yang et al., 2015), long-term performance of high-level nuclear waste repositories (Kim et al., 2011; Neuzil, 2013), enhanced geothermal systems (Huenges, 2016; Zhang et al., 2021), efficient development of unconventional oil and gas resources (Hu et al., 2015; Javadpour, 2009), long-term sealing for carbon utilization and storage (Fakher et al., 2020; Khosrokhavar, 2016), and high-volume and effective gas (hydrogen) storage (Liu et al., 2015; Tarkowski, 2019). For fractured rocks, the accurate characterization of rock matrix and its permeability is also critical for evaluating the effectiveness of low-permeability media, particularly when transport is dominated by slow processes like diffusion (Ghanbarian et al., 2016; Hu et al., 2012).

Standard permeability test procedures in both steady-state and pulse-decay methods use consolidated cm-sized core-plug samples, which may contain fractures and show dual- or triple-porosity characteristics (Abdassah and Ershaghi, 1986; Bibby, 1981). The overall permeability may therefore be controlled by a few bedding-oriented or cross-cutting fractures, even if experiments are conducted at reservoir pressures (Bock et al., 2010; Gensterblum et al., 2015; Gutierrez et al., 2000; Luffel et al., 1993). Fractures might be naturally- or artificially-induced (e.g., created during sample processing), which makes a comparison of permeability results among different samples difficult (Bock et al., 2010; Gensterblum et al., 2015; Gutierrez et al., 2000; Luffel et al., 1993). Hence, methods for measuring the matrix (non-fractured) permeability in tight media, with a practical necessity of using granular samples, have attracted much attention to eliminate the sides effect of fractures (Civan et al., 2013; Egermann et al., 2005; Heller et al., 2014; Wu et al., 2020; Zhang et al., 2020).

A GRI (Gas Research Institute) method was developed by Luffel et al. (1993) and followed by Guidry et al. (1996) to measure the matrix permeability of crushed mudrocks (Guidry et al., 1996; Luffel et al., 1993). Such a method makes permeability measurement feasible in tight and ultra-tight rocks (with permeability < 10-20 m2 or 10 nano-dcarcies, nD), particularly when permeability is close to the detection limit of the pulse-decay approach on core plugs at ~10 nD (e.g., using commercial instrument of PoroPDP-200 of CoreLab). In the GRI method, helium may be used as the testing fluid to determine permeability on crushed samples at different sample sizes (e.g., within the 10-60 mesh range, which is from 0.67 mm to 2.03 mm). The limited mesh size of 20-35 (500-841 μm in diameter) was recommended in earlier works, which has led to the colloquial names of "the GRI method/size" in the literature (Cui et al., 2009; Kim et al., 2015; Peng and Loucks, 2016; Profice et al., 2012). However, Luffel et al. (Guidry et al., 1996; Luffel et al., 1993) did not document the processing methodologies needed to derive the permeability from experimental data from such a GRI method. That is, there are neither standard experimental procedures for interpreting gas pulse-decay data in crushed rock samples nor detailed mathematical solutions available for data processing in the literature (Kim et al., 2015; Peng and Loucks, 2016; Profice et al., 2012). In this work, we achieve to: (1) develop mathematical solutions to interpret gas pulse-decay data in crushed rock samples without published algorithm available as this method shares different constitutive phenomena to the traditional pulse-decay method for core plug samples in Cartesian coordinates; and (2) present associated experimental methodology to measure permeability, reliably and reproducibly, in tight and ultra-tight granular media.

We first derive the constitutive equations for gas transport in granular (unconsolidated or crushed rock) samples. Specifically, we develop three mathematical solutions which cover different experimental situations and sample properties. As each solution shows its own pros and cons, we then in detail present the error analyses for the derived exact and approximate solutions and discuss their applicable requirements and parameter recommendation for practical usages. This work aims to fill the knowledge gap of the granular rock (matrix) permeability measurement and follow-on literature by establishing an integrated methodology for reproducible measurements of nD-level permeability in tight rock for emerging energy and resources subsurface studies.

# 2. Mathematical solutions for gas permeability of granular samples

For a compressible fluid under unsteady-state conditions, flow in a porous medium can be expressed by the mass conservation equation:

∂p∂t+∇·(ρv)=0

where p is the pressure, t is the time, ρ is the fluid density, and v is the Darcy velocity. In continuity equations derived for gas flow in porous media, permeability can be treated as a function of pressure through the ideal gas law. Constitutive equations are commonly established for a small pressure variation to avoid the non-linearity of gas (the liquid density to be a constant) and to ensure that pressure would be the only unknown parameter (Haskett et al., 1988). For spherical coordinates of fluid flow in porous media, assuming flow along the radial direction of each spherical solid grain, Eq.  becomes

∂p∂tϕ=1ctkμr2∂∂r(r2∂p∂r)

The gas compressibility ct is given by

ct=1ρdρdp=1p-1zdzdp

In Eqs. (1B) and (1C), ϕ and k are sample porosity and permeability, r is the migration distance of fluid, μ is the fluid viscosity, and z is the gas deviation (compressibility) factor and is constant.

To correct for the non-ideality of the probing gas, we treat gas density as a function of pressure and establish a relationship between the density and the permeability through a pseudo-pressure variable (given in the 1st part of Supplemental Information SI1). Detailed steps for deriving mathematical solutions for the GPT can be found in SI2, based on heat transfer studies (Carslaw and Jaeger, 1959). The Laplace transform, in combination with the Bessel equation, is an efficient tool for solving gas transport in granular samples with low permeabilities, as applied in this study. Alternatively, other approaches, such as the Fourier analysis, Sturm-Liouville method, or Volterra integral equation of the second form may be used (Carslaw and Jaeger, 1959; Haggerty and Gorelick, 1995; Ruthven, 1984).

We applied dimensional variables to derive the constitutive equation given in Eq. (S10) for which the initial and boundary conditions are

∂2Us∂ξ2+s2Us=0Us=0, ξ=0

α2(Us-1)=3Kc(∂Us∂ξ-Usξ)ξ=1

where Us and ξ represent the dimensionless values of gas density and sample scale, and s is the transformed Heaviside operator. α in Eq. (2B) is determined by solving Eq. (S30) for its root. Kc in Eq.  is a critical parameter that represents the volumetric ratio of the total void volume of the sample cell to the pore volume of the porous samples. It is similar to the storage capacity, controlling the acceptable measurement range of permeability and decay time, in the pulse-decay method proposed by Brace et al. (1968).

The fractional gas transfer for the internal (limited Kc value) and external (infinite Kc value) gas transfer of sample is given by

Ff=1-6n=1∞Kc(1+Kc)e-αn2τ9(Kc+1)+αn2Kc2

Fs=1-6π2n=1∞e-(nπ)2τn2

where Ff and Fs represent the uptake rate of gas outside and inside the sample separately as a dimensionless parameter, and τ is the Fourier number of dimensionless time. Three approximate solutions of the transport coefficient based on Eqs. (2C) and (2D) for various conditions are presented below.

The late-time solution to Eq.  for a limited Kc value (called LLT hereafter) is

k=Ra2μctϕfs1α12

The late-time solution to Eq.  when Kc tends to infinity (ILT hereafter) is

k=Ra2μctϕfs2π2

The early-time solution to Eq.  when Kc approaches infinity (IET hereafter) is

k=πRa2μctϕfs336

In Eq. (3), Ra is the particle diameter of a sample, and s1, s2, and s3 are the three exponents that may be determined from the slopes of data on double logarithmic plots. Table 1 summarizes Eqs. (3A) to (3C) and conditions under which such approximate solutions would be valid.

#### Table 1. Solutions schematic with difference Kc and τ values

| Parameter | Symbol | Remarks |  |  |
| --- | --- | --- | --- | --- |
| Volume fraction§ | Kc | Limited value for Kc < 10 | Infinity value for Kc > 10 |  |
| Exact. Density fraction£ | F | Ff | Fs |  |
| Approx. Solution of Density fraction* | Eqs. (3A-3B) | Eq. (3A) (LLT) | Eq. (3C) (IET) | Eq. (3B) ) (ILT) |
| Available Dimensionless time for Approx. solution | τ | Late-time solution τ> 0.024 | Early-time solution τ< 0.024 | Late-time solution τ> 0.024 |
| § It defines as the volumetric ratio of the total void volume of the sample cell to the pore volume of the porous samples, the classification between the limited and infinity value is proposed as 50 with the following analyses. |  |  |  |  |
| £ The original constitutive equation for different Kc value. |  |  |  |  |
| * Eqs. (3A-3C) are three approximate solutions of density faction function F. |  |  |  |  |

Based on diffusion phenomenology, Cui et al. (2009) presented two mathematical solutions similar to our Eqs. (3A) and (3C). In the work of Cui et al. (2009), however, the lack of detailed analyses of τ and  Kc in the constitutive equations may deter the practical application of Eq. (3B), which is unable to cover an experimental condition of small sample mass with a greater τ (further analyzed in section 3). In addition to that, Cui et al. (2009) did not comprehensively assess practical applications of their two solutions, which is addressed in this study. Hereafter, we refer to the developed mathematical and experimental, gas-permeability-measurement approach holistically as gas permeability technique (GPT).

# 3. Practical usages of algorithms for the GPT

As aforementioned, mathematical solutions given in Eqs. (3A) and (3B) were deduced based on different values of Kc  and τ as shown in the SI2. This means each solution holds only under specific experimental conditions, which are mostly determined by the permeability, porosity, and mass of samples, as well as gas pressure and void volume of the sample cell. In this section, the influence of parameters Kc and τ on the solution of constitutive equation is analyzed and a specific value of dimensionless time (τ = 0.024) is proposed as the criterion required to detect the early-time regime from the late-time one for the first time in the literature. We also demonstrate that the early-time solution of Eq. (3C), which has been less considered for practical applications in previous studies, is also suitable and unique under common situations. Besides, the error of the approximate solution compared to the exact solution and their capabilities are discussed, as it helps to select an appropriate mathematical solution at small τ values. Moreover, we showcase the unique applicability and feasibility of the new solution of Eq. .

### 3.1 Sensitivity analyses of the Kc value for data quality control

To apply the GPT method, appropriately selecting the parameter Kc in Eqs.  - is crucial, as it is a critical value for data quality control. KcThe dimensionless density outside the sample, Uf, is related to Kc via Eq.  in the SI2. One may simplify Eq. (S33) by replacing the series term with some finite positive value and set

Uf-Kc1+Kc>0

We define Kf=Kc/(1+Kc) to interpret the density variance of the system as Kf is closely related to the dimensionless density outside the sample, Uf.

Eq.  shows the relationship between the Uf and Kc (Fig. 1). For Kc > 0, Kf falls between 0 and 1. The greater the Kf value is, the insensitive to density changes the system would be. For Kc equal to 50 (not shown in Fig. 1), Kf would no longer be sensitive to Kc variations as it has already approached 98% of the dimensionless density. This means that the Uf value needs to be greater than 0.98, and this leaves only 2% of the fractional value of Uf available for capturing gas density change. When Kc is 100, the left fractional value of Uf would be 1%. This would limit the amount of data available (the linear range in Fig. S1) for the permeability calculation, which would complicate the data processing. Thus, for the GPT experiments, a small value of Kc (less than 10) is recommended, as Kf nearly reaches its plateau beyond Kc = 10 (Fig. 1). When Kc is 10, the left fractional value of Uf is only as low as 9%.

#### Fig. 1. Dimensionless density, Kf, as a function of dimensionless volume Kc. Recall that Kf=Kc/(1+Kc). Major variations in Kf occur for Kc < 10 indicating longer gas transmission duration with more pressure-decay data available for permeability derivation.

### 3.2 Recommendation for solution selection

The following three aspects need to be considered before selecting the appropriate solution for permeability calculation: 1) selecting early- or late-time solutions; 2) error between the approximate and exact solutions; and 3) the convenience and applicability of solutions suitable for different experiments. We will first discuss the selection criteria for early- or late-time solutions.

Fig. 2(a) shows the exact solution of Fs with their two approximate early- and late-time solution (Table 1). Two exact solutions of Ff where Kc equals to 10 or 50 are also demonstrated in Fig. 2(a). Fig. 2(b) depicts the exact solution from Ff for different Kc values from 1 to 100 and their corresponding approximate solution for Eq. (3A). The intersection point of the solution Eq.  and Eq. (3C), namely τ = 0.024 in Fig. 2(a), is used for distinguishing early- and late-time solutions.

Two notable observations can be drawn from Fig. 2(b). Firstly, the approximate solution Eq.  would only be applicable at late times when τ is longer than 0.024. For τ< 0.024, regardless of the Kc value, Eq.  would be more precise than Eqs. (3A) and (3B) and return results close to the exact solution for both Ff and Fs. Secondly, results of Eqs. (3A) and  presented in Fig. 2(a) are similar; Kcthere difference is very small especially for Kc> 10. Due to the fact that core samples from deep wells are relatively short in length and their void volume is small (ultra-low porosity and permeability such as in mudrocks with k≤ 0.1 nD), in practice, a solution for 10<Kc<100 is the most common outcome, even if the sample cell is loaded as full as possible. Under such circumstances, the newly derived solution, Eq. , becomes practical and convenient: 1) if the Kc and dimensionless time τ have not been evaluated precisely before the GPT experiment, this solution may fit most experimental situations; 2) this solution is suitable for calculation as it does not need the solution from the transcendental equation of Eq.  because the denominator of α has been replaced by π. The data quality control is discussed in Section 4.1.

#### Fig. 2. Three GPT solutions with different values of τ, Kc; the dashed lines are approximate solutions without a series expansion in Fig. (2b) for Ff. Figure modified from Cui et al. (2009).

### 3.3 Applicability of the early-time solution

A small Kc value can guarantee a sufficient time for gas transfer in samples and provide enough linear data for fitting purposes. We note that the selection of the limited Kc solution of Ff, and the infinity Kc solution Fs is controlled by Kc. However, before the selection of Kc, the dimensionless time is the basic parameter to be estimated as a priori before the early- or late-time solutions are selected.

For pulse-decay methods, the early-time solution has the advantage of capturing the anisotropic information contained in reservoir rocks (Jia et al., 2019; Kamath, 1992). However, it suffers from the shortcoming of uncertainty in data for initial several seconds, which as a result is not recommended for data processing (Brace et al., 1968; Cui et al., 2009). This is due to: (1) the Joule-Thompson effect, which causes a decrease in gas temperature from the expansion; (2) kinetic energy loss during adiabatic expansion; and (3) collision between molecules and the container wall. These uncertainties normally occur in the first 10-30 sec, shown in our experiments as a fluctuating period called "Early Stage".

However, the "Early Stage" present in pulse-decay experiments does not mean that the early-time solution is not applicable. We demonstrate the relationship between time and dimensionless time in Fig. 3 that a short dimensionless time may correspond to a long testing period of hundred to thousand seconds in experiments. This is particularly noticeable for the ultra-low permeability samples with k≤ 0.1 nD and small dimensionless times τ < 0.024. This situation would only be applicable to early-time solution, but with data available beyond the "Early Stage" and provide available data in a long time (hundreds to thousands of seconds). For example, the early-time solution would fit ultra-low permeability samples in 600s for 0.1 nD, and at least 1000s for 0.01 nD shown in Fig. 3 in the region below the dark line. Then, using Eq. (3C), the derived permeability would be closer to its exact solution in the earlier testing time (but still after the "Early Stage"). The mudrock samples that we tested, with results presented in Section 5.3, exhibit low permeabilities, approximately on the order of 0.1 nD..

#### Fig. 3 Dimensionless time τ versus actual times for different permeability values trough Eq. (S14) using He gas, sample porosity of 5%, and sample diameter of 2 mm.

### 3.4 Error analyses between exact and approximate solutions

It is unpractical to use the exact solutions with their series part to do the permeability calculation; thus, only the approximate solutions are used and the error difference between the exact and approximate solutions is discussed here. The original mathematical solutions, Eqs.  and , are based on series expansion. For dimensionless densities Ff and Fs in Eqs.  and , their series expansion terms should converge. However, the rate of convergence is closely related to the value of τ. For example, from Eq. , when τ≥1, the exponent parts of Us and Uf are at least (2n+1)π2. Therefore, the entire series expansion term can be omitted without being influenced by Kc. In practical applications, the solutions given in Eqs. - are approximates without series expansion. In this study, we provide the diagrams of change in errors with dimensionless time in the presence of adsorption (Fig. 4).

For Ff, the error differences between the exact and approximate solutions are 3.5% and 0.37% for τ = 0.05 and 0.1 when Kc = 10, respectively. When τ≤ 0.024, the error would be greater than 14.7%. Fig. 2(b) shows that Ff can be approximated as Fs when Kc is greater than 10; the error difference between Ff and Fs is quite small at this Kc value (for Kc = 10, 6.6% is the maximum error when τ = 0.01; 4.4% when τ = 0.05; and 2.9% when τ = 0.1) as shown in Fig. 4.

For Fs, the error difference is roughly the same as Ff and equal to 3.6% for τ = 0.05 and 0.38% for τ = 0.1. This verifies that newly derived Eq.  is equivalent to Eq.  when Kc is greater than 10. As for the evaluation of Eq. , the error difference with the exact solution will increase with dimensionless time (5.1% for τ = 0.003, 9.7% for τ = 0.01, and 16% for τ = 0.024).

#### Fig. 4. Error analyses of Ff and Fs for their exact and approximate solutions

In the following, we apply the approximate solutions, Eqs. (3A-3C), to some detailed experimental data and determine permeability in several shale samples practically compatible with sample size, gases, and molecular dynamics analyses.

# 4. Influence of kinetic energy on gas transport behavior

### 4.1 Flow state of gas in granular samples

In the following, we apply the approximate solutions, Eqs. (3A-3C), to some detailed experimental data and determine permeability in several mudrock samples practically compatible with sample size, gases, and molecular dynamics analyses.

During the GPT, with the boundary conditions described in SI2, the pressure variation is captured after gas starts to permeate into the sample from the edge, and the model does not take into account the gas transport between particles or into any micro-fractures, if available. Thus, the transport that conforms to the "unipore" model and occurs after the "Early Stage" (defined in Section 3.3) or during the "Penetration Zone" (the area between the two vertical lines in Fig. 5), should be used to determine the slope. Fig. S2 shows how to obtain the permeability result using the applicable mathematical solutions (Eqs. 3A-C). Fig. 5 shows the pressure variance with time during the experiment using sample size from 0.34 mm to 5.18 mm for sample X-1 and sample X-2. From Fig. 5, the time needed to reach pressure equilibrium after the initial fluctuation stage is 20-100 sec, and the “Penetration Zone” decreases with decreasing grain size over this time period.

#### Fig. 5. Fitting region (the "Penetration Zone" in the shadowed area) for mudrock sample X-1 with different granular sizes; the penetration zone illustrating the pressure gradient mainly happens at 20 to 200 sec for this sample.

In fact, the "Penetration Zone", as an empirical period, is evaluated by the pressure change over a unit of time before gas is completely transported into the inner central part of the sample to reach the final pressure. Owing to the sample size limitation, a decreasing pressure could cause multiple flow states (based on the Knudsen number) to exist in the experiment. The pressure during the GPT experiment varies between 50 and 200 psi (0.345 MPa to 1.38 MPa). Fig. 6 shows the Knudsen number calculated from different pressure conditions and pore diameters together with their potential flow state. Based on Fig. 6, the flow state of gas in the GPT experiments is mainly dominated by Fickian and transition diffusion. Essentially, the flow state change with pressure should be strictly evaluated through the Knudsen number in Fig. 6 to guarantee that the data in the "Penetration Zone" are always fitted with the GPT's constitutive equation for laminar or diffusive states. This helps obtain a linear trend for  ln(1-Ff) or Fs2 versus time for low-permeability media. Experimentally, data from 30 to several 100 seconds are recommended for tight rocks like shales within the GPT methodology.

#### Fig. 6. Flow state of gas under diffferent testing pressures; modified from Chen & Pfender (1983) and Roy et al. (2003) (Chen and Pfender, 1983; Roy et al., 2003).

In the GPT approach, as mentioned earlier, Eq.  holds for small Kc values (e.g., < 10) so that the approximately equivalent void volume in the sample cell and sample pore volume would allow for sufficient pressure drop. It also gives time and allows the probing gas to expand into the matrix pores to have a valid "Penetration Zone" and to determine the permeability. Greater values of Kc would prevent the gas flow from entering into a slippage state as the pressure difference would increase with increasing Kc. However, large pressure changes would result in a turbulent flow (Fig. 6), which would cause the flow state of gas to be no longer valid for the constitutive equation of the GPT. Overall, the GPT solutions would be applicable to the gas permeability measurement, based on the diffusion-like process, from laminar flow to Fickian diffusion, after the correction of the slippage effect.

### 4.2 Pressure decay behavior of four different probing gases

We used three inert gases, including He, N2, and Ar, and one sorptive gas i.e., CO2 (Busch et al., 2008), to compare the pressure drop behavior for sample size with an average granular diameter of 0.675 mm. Results for the mudrock sample X-2 are presented in Fig. 7. Among the three inert gases, helium and argon required the shortest and longest time to reach pressure equilibrium (i.e., He<N2<Ar). In terms of pressure drop, argon exhibited the most significant decrease. In a constant-temperature system, the speed (or rate) at which gas molecules move is inversely proportional to the square root of their molar masses. Hence, it is reasonable that helium (with the smallest kinetic diameter of 0.21 nm) has the shortest equilibrium time. However, the pressure drop is more critical than the time needed to reach equilibrium for the GPT, as the equilibrium time does not differ much (basically within 10 seconds for a given sample weight, except for the adsorptive CO2). Argon may provide a wider range of valid Penetration Zones in a short time scale for its longest decay time except for adsorbed gas of CO2; a choice of inert and economical gas is suggested for the GPT experiments.

#### Fig. 7. Measured pressure decay curves from mudrock Sample X-2 for gases of different molecular diameters σ and molecular weights M (g/mol).

Fig. 7 shows that the pressure decay curve of the adsorptive gas CO2 is different from those of the inert gases used in this study. CO2 has a slow equilibrium process due to its large molar mass, and the greatest pressure drop among the four gases due to its adsorption effect. This additional flux needs to be taken into account to obtain an accurate transport coefficient. Accordingly, multiple studies including laboratory experiments (Pini, 2014) and long-term field observations (Haszeldine et al., 2006; Lu et al., 2009) were carried out to assess the sealing efficiency of mudrocks for CO2 storage. In fact, the GPT can supply a quick and effective way to identify the adsorption behavior of different mudrocks for both laminar-flow and diffusion states.

### 4.3 Pressure decay behavior for different granular sizes

We compared the pressure drop behavior of gas in the mudrock Sample X-1 with different granular sizes (averaged from 0.34 mm to 5.18 mm) using the same sample weight and Kc. Results based on the experimental data shown in Fig. 8 indicate that a larger-sized sample would provide more data to be analyzed for determining the permeability. This is because the larger the granular size, and (1) the larger the pressure drop, (2) the longer the decay time as Fig. 8 demonstrates. This is consistent with the simulated results reported by Profice et al. (2012).

#### Fig. 8. Pressure decay curves measured by helium on sample X-1 with five different granular sizes. The intra-granular porosity was 5.8% independently measured by mercury intrusion porosimetry.

#### Table 2. Permeability results from the methods of GPT and SMP-200 for X-1.

| Granular size (mm) | SMP-200 (nD) § | GPT test 1 (nD)£ | GPT test 2 (nD)£ | Average value (nD)£ | Fitting duration (s) | Solution type | Dimensionless time | Particle density (g/cm3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.18 | - | 1.17 | 1.17 | 1.17 | 50-100 | ILT | 0.023-0.027 | 2.631 |
| 2.03 | 14.2 | 0.45 | 0.41 | 0.43 | 50-100 | LLT | 0.026-0.028 | 2.626 |
| 1.27 | - | 0.10 | 0.10 | 0.10 | 30-60 | ILT | CR* | 2.673 |
| 0.67 | 0.65 | 0.08 | 0.04 | 0.06 | 30-60 | LLT | CR* | 2.658 |
| 0.34 | - | 0.02 | - | 0.02 | 30-60 | IET | CR* | 2.643 |
| § The results are from the SMP-200 using the GRI default method. |  |  |  |  |  |  |  |  |
| £ The results are from the GPT method we proposed. |  |  |  |  |  |  |  |  |
| * CR means the conflict results that the verified dimensionless time does not confirm the early- or late-time solutions using the solved permeability. For example, the verified dimensionless time would be > 0.024 using the early-time solution solved result and vice versa. |  |  |  |  |  |  |  |  |

As reported in Table 2, the permeability values measured by the GPT method are one or two orders of magnitude greater than those measured by the SMP-200 instrument. The built-in functions of SMP-200 can only be used for two default granular sizes (500-841 μm for GRI and 1.70-2.38 mm for what we call GRI+) to manually curve-fit the pressure decay data and determine the permeability. The GRI method built in the SMP-200 only suggests the fitting procedure for data processing without publicly available details of underlying mathematics. The intra-granular permeabilities of mudrocks sample X-1 vary from 0.02 to 1.17 nD for five different granular sizes using the GPT. With the same pressure decay data selected from 30 to 200 sec, the permeability results for GRI and GRI+ sample sizes from the SMP-200 fitting are 0.65 and 14.2 nD, as compared to 0.06 and 0.43 nD determined by the GPT using the same mean granular size. Our results are consistent with those reported by Peng & Loucks (2016) who found two to three orders of magnitude differences between the GPT and SMP-200 methods (Peng and Loucks, 2016).

There exist several issues associated with granular samples with diameters smaller than on average 1.27 mm. First, the testing duration is short, and second, there would not be sufficient pressure variation analyzed in Fig. 8. Both may cause significant uncertainties in the permeability calculation and, therefore, make samples with diameters smaller than 1.27 mm unfavorable for the GPT method, particularly extra-tight (sub-nD levels) samples, as there is almost no laminar or diffusion flow state to be captured. The greater pressure drop for larger-sized granular samples would result in greater pressure variation and wider data region compared to smaller granular sizes (see Figs. 6 and 9). Although samples of large granular sizes may potentially contain micro-fractures, which complicate the determination of true matrix permeability (Heller et al., 2014), the versatile GPT method can still provide size-dependent permeabilities for a wide range of samples (e.g., from sub-mm to 10 cm diameter full-size cores) (Ghanbarian, 2022a, b). Besides, the surface roughness of large grains may also complicate the determination of permeability, which need to pay attention to (Devegowda, 2015; Rasmuson, 1985; Ruthven and Loughlin, 1971). Overall, our results demonstrated that sample diameters larger 2 mm are recommended for the GPT to determine the nD permeability of tight mudrocks, while smaller sample sizes may produce uncertain results.

### 4.4 Practical recommendations for the holistic GPT

Here, we evaluate the potential approximate solution for tight rock samples using frequently applied experimental settings by considering the critical parameters, such as sample mass, porosity, and estimated permeability (as compiled in Fig. 9 showing the dimensionless time versus porosity). Based on the results presented in Figs. 3 and 6, only t < 200s is dominant and critical for the analyses of dimensionless time and penetration zone. Thus, we take 200s and use helium to calculate the dimensionless time. Another critical parameter to assure enough decay data is the sample diameter greater than 2 mm. Thus, we only show the dimensionless time versus porosity for sample diameter greater than the criteria of 2 mm.

Fig. 9 demonstrates that the sample permeability has dominant control on the early- or late-solution selection, and we decipher a concise criterial for three solutions selection. We classify the dimensionless time versus porosity relationship into three cases. Firstly, among the curves shown in Fig. 9, only that corresponding to k = 0.1 nD and sample diameter of 2 mm stays below the dashed line representing τ = 0.024. Therefore, the early time solution is appropriate for tight samples with permeabilities less than 0.1 nD (as shown in the analyses of Section 4.3, which also conforms to the situation of the molecular sieve sample that we tested in SI3). Secondly, for permeabilities greater than 10 nD (the curve is above the line of τ = 0.024), the new derived late-time solution, Eq. (3B), is recommended as it is more convenient for mathematical calculation without the consideration of transcendental functions. The reason is that the sample cell can be filled as much as possible (~90% of the volume) with samples and solid objects. However, as the tight rock hardly presents a large value of porosity, the small Kc value is difficult to be achieved with an inconsequential influence between Eq. (3B) and Eq. (3A). Lastly, in the case of permeability around 1 nD, the value of porosity would be critical in the selection of the early- or late-time solutions, as shown in Fig. 9.

#### Fig. 9. Holistic GPT to explore the appropriate solution based on diameter, permeability, and porosity of samples. The legend shows the diameter of granular sample and permeability, along with a dashed line for dimensionless time of 0.024, while regions above and below this value fit for the late- and early-time solutions, respectively.

# 5. Conclusions

In the present work, we solved fluid flow state equation in granular porous media and provided three exact mathematical solutions along with their approximate ones for practical applications of low permeability measurements. The mathematical solutions for the transport coefficient in the GPT were derived for a spherical coordinate system, applicable from laminar flow to slippage-corrected Fickian diffusion. Of the three derived solutions, one is valid during early times when the gas storage capacity Kc approaches infinity, while the other two are late-time solutions to be valid when Kc is either small or tends towards infinity. KcKcKcWe evaluated the derived solutions for a systematic measurement of extra-low permeabilities in granular media and crushed rocks using experimental methodologies with the data processing procedures. We determined the error for each solution by comparing with the exact solutions presented in the SI. The applicable conditions for such solutions of the GPT were investigated, and we provided the selection strategies for three approximate solutions based the range of sample permeability. In addition, a detailed utilization of GTP was given to build up the confidence in the GPT method through the molecular sieve sample, as it enables a rapid permeability test for ultra-tight rock samples in just tens to hundreds of seconds, with good repeatability.

# Data availability. This work did not use any data from previously published sources, and our experimental data & processing codes of MATLAB are available at https://doi.org/10.18738/T8/YZJS7Y, managed by Mavs Dataverse of the University of Texas at Arlington.

# Supplement. An early-version preprint of this work appears as DOI: 10.1002/essoar.10506690.2 (Zhang et al., 2021).

Author contributions. TZ and QHH planned and designed the research, performed the analyses, and wrote the paper with contributions from all co-authors. BG, DK, and ZM participated in the research and edited the paper.

# Competing interest. We declare that we do not have any commercial or associative interest that represents a conflict of interest in connection with the work submitted.

# Acknowledgments. Financial assistance for this work was provided by the National Natural Science Foundation of China (41830431; 41821002), Shandong Provincial Major Type Grant for Research and Development from the Department of Science & Technology of Shandong Province (2020ZLYS08), Maverick Science Graduate Research Fellowship for 2022-2023, and Kansas State University through faculty start-up funds to BG.

# Nomenclature

Bij Correction parameter for viscosity, constant

ct Fluid compressibility, Pa-1

D Diffusion coefficient, m2/s

Ff Uptake rate of gas outside the sample, dimensionless

Fs Uptake rate in the sample, dimensionless

f1 Intercept of Eq. , constant

J Physical flux, unit for certain physical phenomenon

Ka Apparent transport coefficient defined as Eq. , m2/s

Kc Ratio of gas storage capacity of the total void volume of the system to the pore (including adsorptive and non-adsorptive transport) volume of the sample, fraction

Kf Initial density state of the system, fraction

k Permeability, m2

ks Permeability defined as Eq. , m2/(pa⋅s)

L Coefficient, unit for certain physical transport phenomenon

M Molar mass, kg/kmol

Mm Molar mass of the mixed gas, kg/kmol

Mi,j Molar mass for gas i or j, kg/kmol

Ms Total mass of sample, kg

N Particle number, constant

p Pressure, Pa

pcm Virtual critical pressure of mixed gas, Pa

pp Pseudo-pressure from Eq. , Pa/s

Ra Particle diameter of sample, m

R Universal gas constant, 8.314 J/(mol⋅K)

r Diameter of sample, m

s1 Slope of Eq. , constant

s2 Slope of function Ln(1-Fs), constant

s3 Slope of function Fs2, constant

T Temperature, K

Tcm Virtual critical temperature for mixed gas, K

t Time, s

Uf Dimensionless density of gas outside the sample, dimensionless

Us Dimensionless density in grain, dimensionless

U∞ Maximum density defined as Eq. , dimensionless

V1 Cell volume in upstream of pulse-decay method, m3

V2 Cell volume in downstream of pulse-decay method, m3

Vb Bulk volume of sample, m3

Vc Total system void volume except for sample bulk volume, m3

v Dacian velocity in pore volume of porous media, m/s

X Pressure force, Pa

yi,j Molar fraction for gas i or j, fraction

z Gas deviation (compressibility) factor, constant

Greek Letters:

αn The nth root of Eq. , constant

μ Dynamic viscosity, pa⋅s or N⋅s/m2

μi,j Dynamic viscosity for gas i or j, pa⋅s or N⋅s/m2

μmix Dynamic viscosity of mixture gas, pa s or N s/m2

μp Correction term for the viscosity with pressure, pa s or N s/m2

ξ Dimensionless radius of sample, dimensionless

ρ Density of fluid, kg/m3

ρ0 Average gas density on the periphery of sample, kg/m3

ρ1 Gas density in reference cell, kg/m3

ρ2 Gas density in sample cell, kg/m3

ρb Average bulk density for each particle, kg/m3

ρf Density of gas changing with time outside sample, kg⋅m-3⋅s-1

ρf∞ Maximum value of ρf defined as Eq. , kg⋅m-3⋅s-1

ρp Pseudo-density from Eq. , kg⋅m-3⋅s-1

ρs Density of gas changing with time in sample, kg⋅m-3⋅s-1

ρps Pseudo-density of gas changing with time in sample, kg⋅m-3⋅s-1

ρpf Pseudo-density of gas changing with time outside sample, kg⋅m-3⋅s-1

ρp2 Initial pseudo-density of gas in sample, kg⋅m-3⋅s-1

ρp0 Average pseudo-density of gas on sample periphery, kg⋅m-3⋅s-1

ρrm Relative density to the mixed gas, kg⋅m-3⋅s-1

ρsav Average value ofρsrdefined as Eq. , kg⋅m-3⋅s-1

ρsr Average value of pseudo-density of sample changing with diameter, kg⋅m-3⋅s-1

ρs∞ Maximum value of ρsr defined as Eq. , kg⋅m-3⋅s-1

τ Dimensionless time, dimensionless

ϕ Sample porosity, fraction

ϕf Total porosity (ϕf=ϕa+ϕb) occupied by both free and adsorptive fluids, fraction

# Supporting Information (SI)

### SI1. Consideration of Non-linearity of Gas and Solutions for a Mixed Gas State

For gas flow, we can use a pseudo-pressure variable to linearize Eq. (1D) as μ and ct are functions of pressure. Thee pseudo-pressure pp is defined as (Haskett et al., 1988)

pp=2p0ppμzdp

By combining Eq.  with the ideal gas law, the pseudo-density may be expressed as

ρp=pMRT=p2MμzRT

Because viscosity and compressibility do not change significantly (less than 0.7%) between 200 psi and atmospheric pressures, Eq.  can be simplified to

ρp=p2MRT

Thus, the density change is replaced by the pseudo-density for a precise calibration by using pressure squared.

During the GPT experiment, different gases in the reference and sample cells may complicate the hydrodynamic equilibrium of gas, and consequently the expression of transport phenomena, as the viscosity and gas compressibility are in a mixed state. Therefore, during the GPT experiment when a different gas exists between the reference and sample cells a, a mixed viscosity should be used after the gas in reference cell is released into the sample cell. The viscosity of mixture μmix under pressure in Eqs. - can be calculated from (Brokaw, 1968; Sutherland, 1895)

μmix=μi1+1yi(&j=1&j≠inBijyj)+μp

Bij is a correction parameter independent of gas composition and can be expressed as

Bij=[1+(μiμj)0.5(MjMi)0.5]222(1+MjMi)0.5

in which μp is the correction term for the viscosity variation as its changes with pressure and given by

μp=1.1×10-8(e1.439ρrm-e-1.111ρrm1.858)×Mm0.5⋅Pcm2/3Tcm1/6

## SI2. Gas Transport in GPT

From Eq. (1D), the transport of gas in the GPT with the "unipore" model under a small pressure gradient in a spherical coordinate system with laminar flow is based on the Darcy-type relation. Because the transfer rate of the fluid is proportional to the concentration gradient, this process can be expressed as:

∂ρp∂t=kctϕfμ(2r∂ρp∂r+∂2ρp∂r2)

We set

ks=kμ

Ka=ksctϕf

Then, Eq.  becomes:

∂ρp∂t=Ka(2r∂ρp∂r+∂2ρp∂r2)  or  ∂∂t(ρpr)=Ka∂2∂r2(ρpr)

We next introduce the following dimensionless variables:

Us=rR(ρps-ρp2)(ρp0-ρp2)

Uf=ρpf-ρp2ρp0-ρp2

ξ=rR

τ=KatR2

where ρ1 and ρ2 are the gas density in the reference and sample cells, and ρ0 is the gas density outside the connected pore volume (the gas has flowed from the reference into sample cells but not into samples), and ρ0 is given by

ρ0=V1ρ1+(V2-Vb)ρ2Vc

where V1 is the reference cell volume, V2 is the sample cell volume, Vb is the bulk volume of the sample, Vc is the total void volume of the system minus Vb where Vc=V1+V2-Vb.

If the bulk density of the sample is ρb and the total mass of the sample is Ms, then the total number of sample particles N is:

N=Ms43πRa3ρb

Based on Darcy's law, the gas flow into a sample Q is:

Q=-4πR2(ks∂p∂r)N=-3RMsρbks∂p∂r

According to mass conservation and in combination with Eq. , for t>0 and r=Ra, we have

-3RVbKactϕf∂p∂rρs=Vc∂ρf∂t

Substituting Eq.  into Eq. , the boundary condition of Eq. , for ξ =1, is:

-3RVbKaϕf∂ρs∂r=Vc∂ρf∂t

Substituting dimensionless variables into Eq.  yields:

∂Us∂τ=∂2Us∂ξ2

By defining parameter Kc as:

Kc=VcVbϕf

the boundary condition of Eq.  becomes:

∂Uf∂τ=-3Kc(∂Us∂ξ-Usξ)

From Eq. , Kc represents the ratio of gas storage capacity of the total void volume of system to the pore volume (including both adsorption and non-adsorption volume) of sample.

The initial condition of Eq. , for τ=0, is:

when 0≤ξ<1, Us=0

For τ>0:

ξ=0, Us=0

ξ=1, Us=Uf=1

∂Us∂τ=∂2Us∂ξ2, 0<ξ<1

Replacing the Heaviside operator p=∂/∂τ as p=-s2, Eq.  and Eq.  then become:

∂2Us∂ξ2+s2Us=0Us=0, ξ=0

α2(Us-1)=3Kc(∂Us∂ξ-Usξ)ξ=1

For these first- and second-order ordinary differential equations, we can solve Eqs.  and  as:

Us=α2sinαξ3Kc(sinα-αcosα)+α2sinα

In Eq. , αn are the roots of Eq. :

tanα=3α3+α2Kc

Defining the numerator and denominator of Eq.  as functions fα and F(α), Us can be expressed as:

Us=Fα→0f(α)F(α)+2n=1∞f(αn)αnF'(αn)e-αn2τ

### SI2-1: Solution for the Limited Kc Value

Under the condition of limited Kc value, Eq.  is solved with the boundary condition of 0<ξ<1 at time t, and the gas state on the grain surface is initially at equilibrium with the gas outside. Using the Laplace transform, Eq.  is given as (the Laplace transform part can be found in APPENDIX V of Carslaw & Jaeger, 1959) (Brokaw, 1968; Sutherland, 1895):

Us=ξKcKc+1+6n=1∞sinξαnsinαnKce-αn2τ9(Kc+1)+αn2Kc2

As the pressure transducer detects the pressure in the reference cell, with the boundary condition Uf=Usξ=1, we can calculate Uf as:

Uf=Kc1+Kc+6n=1∞Kce-αn2τ9(Kc+1)+αn2Kc2

For a convenient expression of αn through logarithmic equation, Eq.  can be transformed as:

(1-Uf)(1+Kc)=1-6n=1∞Kc(1+Kc)e-αn2τ9(Kc+1)+αn2Kc2

The left side of Eq.  clearly has a physical meaning for the state of gas transport outside the sample, and we define (1-Uf)(1+Kc) as Ff, which is less than, but infinitely close to, 1. Parameter Ff represents (1) the fraction of final gas transfer of Vc which has taken place by time t, which can be interpreted as the net change in the density of gas at time t to time infinity as Eq. , or (2) as the fractional approach of the gas density to its steady-state in terms of dimensionless variables as Eq. .

Ff=ρp0-ρpfρp0-ρf∞ or

Ff=1-Uf1-U∞=ρp0-ρpfρp0-ρp2(1+Kc)

where for τ→∞, the result of Uf and ρf∞would tend to be the limiting value:

U∞=Us=Ufξ=ξKc1+Kcξ=1

ρf∞=V1ρ1+(V2-Vs)ρ2V1+V2-Vs=Kc1+Kc(ρp0-ρp2)+ρp2

Thus, Eq.  can be expressed as:

Ff=1-6n=1∞Kc(1+Kc)e-αn2τ9(Kc+1)+αn2Kc2

For calculating the permeability, Eq.  can be linearized as a function of time as there are no variables other than the exponential part:

ln(1-Ff)=f1-s1t

where f1 is the intercept for the y-axis of function :

f1=ln[6Kc(1+Kc)9(1+Kc)+α12Kc2]

The slope s1 can be captured by the fitted line of the linear segment, and α1 is the first solution of Eq. :

s1=α12KaRa2

Thus, the permeability can be calculated as:

k=Ra2μctϕfs1α12

### SI2-2: Solution for Kc Goes to Infinity

When Vc has an infinite volume compared to the void volume in a sample, which means that the density of gas in Vc would be kept at ρp0, and α would approach nπ in Eq. , then Eq.  can be transformed as:

Us=ξ+2πn=1∞(-1)nsinnπξne-(nπ)2τ

In this situation, Uf=1, and as the gas density would be maintained at the initial state at ρp0, it would be a familiar case in diffusion kinetics problems with the uptake rate of Ff to be expressed as Fs in Vb (Barrer, 1941):

Fs=ρsavρs∞

where ρsav is the average value of ρsr in the grain, and ρs∞ is the maximum value of ρsr:

ρsr=ρps-ρp2,   ρs∞=ρp0-ρp2

The value of ρsr in the grain is:

ρsav=3R30Rρsrr2dr

Then Fs becomes:

Fs=3R30RUsξr2dr

Substituting Eq.  into Eq. , we can calculate:

Fs=1-6π2n=1∞e-(nπ)2τn2

Similar to Eq. , Eq.  can also be linearized to calculate the permeability in τ from the fitted slope. For τ≥0.08, Eq.  can be reduced as:

Fs=1-6π2e-π2τ

When t is small enough (for τ≤0.002), Eq. can be transformed into Eq. .

Fs=6τπ

As Fs is a special solution of Ff with the case of Kc goes to infinity, we can arrive at:

Fs=Ff=(1-Uf)(1+Kc)

For testing the ultra-low permeability rocks using granular samples whenKc goes to infinity, Eq.  and Eq.  can be selected using different τ values.

From the fitted slope s2 of function ln(1-Fs) from Eq. , we can then derive the permeability:

k=Ra2μctϕfs2π2

The results of Eq.  are very similar to Eq. (S43) as the first solution for Eq.  is very close to π.

From the fitted slope s3 of function Fs2 from Eq. , we can derive the permeability:

k=πRa2μctϕfs336

### SI3. A Case of Data Processing for GPT

We show here an illustration of the data processing procedure for the GPT with a molecular sieve sample (https://www.acsmaterial.com/molecular-sieves-5a.html). This material consists of grains of 2 mm in Diameter with a porosity of 26.28%, and a uniform pore-throat size of 5Å in Diameter, with a particle density of 2.96 g/cm3. For a 45 g sample, the Kc value is 19.4 from Eq. , and therefore 4.9% of the density ratio (1-Kf) is available for mass transfer from Eq. .

The experimental data were captured under a strict temperature control and unitary-gas environment, along with a precise measurement of barometric pressure. The experiment was run twice, and after the data were collected, 1) we made a rough evaluation of the "Penetration Zone" of this sample based on Figs. 5-6. For this molecular sieve sample, the "Penetration Zone" is shown in Fig. S1, and the mass transfer in unit time more conforming to a linear state (shown as Fig. 5) over a large time range, especially at 100-300s; 2) data in the selected range (100-300s) were fitted respectively for the slope from Fig. S2, then slopes were compiled in Table SI3-1; 3) permeabilities were calculated using the slope of the fitted curve, and all results for LLT, ILT and IET are also shown in Table SI3-1; 4) the results were checked with their dimensionless times to verify whether the early- or late-time solutions were used correctly. Table SI3-1 clearly shows that the results of IET should be selected for this sample, as the dimensionless time is less than 0.024. Note that the data fluctuation shown here was from a high resolution (±0.1% for 250 psi) pressure sensor without undergoing a smoothing process; meanwhile, for data in the 100-200, 200-300, and 300-400 seconds of experimental duration, 100, 200, and 300 seconds respectively were used to calculate the dimensionless times for the results in Table SI3-1.

In addition, the validity of the permeability obtained needs to be verified by using the time interval employed in data fitting and the calculated permeability results to calculate the τ (Table SI3-1). If the dimensionless time is less than 0.024 (as occurred for the case of molecular sieve), the IET solution is selected; if the dimensionless time is greater than 0.024 and Kc is greater than 10, the ILT solution is used; if τ is greater than 0.024 and Kc is less than 10, then the LLT solution is employed. However, for sample sizes smaller than 1.27 mm, Conflicting Results (described in Table 1) occur, and results from this situation are not recommended due to poor data quality.

#### Table SI3-1. Permeability results of molecular sieve from LLT, IET and ILT

| Fitting range (s) | LLT (m2) | τ -LLT | IET(m2) | τ -IET | ILT (m2) | τ -ILT | Slope-LLT | Slope-IET | Slope-ILT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100-200 | 5.60E-22 | 0.004 | 1.02E-21 | 0.007 | 5.00E-22 | 0.003 | 0.0004 | 0.0007 | 0.0004 |
| 200-300 | 4.20E-22 | 0.006 | 5.81E-22 | 0.008 | 3.75E-22 | 0.005 | 0.0003 | 0.0004 | 0.0003 |
| 300-400 | 2.80E-22 | 0.006 | 4.36E-22 | 0.009 | 2.50E-22 | 0.005 | 0.0002 | 0.0003 | 0.0002 |

#### Fig. S1. Unit pressure change varying with experimental time.

#### Fig. S2. Fitted slopes for each solution; (a) to (c) are results of LLT and ILT, while (d) to (f) of IET.

### SI4. Equipment and samples

The experimental setup in the GPT presented in this study is based on the GRI-95/0496 protocols (Guidry et al., 1996) and the SMP-200 guidelines from Core Laboratories with the gas expansion approach (shown in Fig. S3). In this work, gases (He, Ar, N2, or CO2) with different molecular sizes and sorption capacities were tested using two shale core samples (X1, X2) from an oil-producing lacustrine formation in the Songliao Basin, China. X1 is used for sample size study where X2 used for experiment with different gas. Also, we used the molecular sieve to exhibit the practical utilization of the GPT method in SI3. We gently crushed the intact samples with mortar and ground to different granular sizes from 0.34 mm to 5.18 mm through a stack of sieves (named here as Size X: 8 mm to #8 mesh; GRI+: #8-#12 mesh; Size A: #12-#20 mesh; GRI: #20-#35 mesh; Size B: #35-#80 mesh).

#### Fig. S3. Scheme of the GPT experiment for granular samples with all the cells and supplies placed inside an incubator for temperature control.

After loading each sample, related accessories (e.g., solid discs or balls for volume control; and hence porosity, sample mass, and solution-related) were placed below samples inside the cell (Fig. S3). Next, valves 1 and 3 were closed, then valves 2 and 4 were opened for air evacuation. Using a precise pressure gauge connected to the reference cell shown in Fig. S3 we monitored changes in the pressure. The evacuation time typically lasted at least 15-30 min, and then the system was allowed to stabilize for another 15 min. As the moisture content of the samples significantly influences the final vacuum, the samples were placed into the sample cell immediately after removal from the drying oven set at 60oC for two days and cooling in a low-humidity desiccator.

The experiments were conducted at the temperature of 35°C by placing the SMP-200 inside an incubator equipped with a high precision temperature-humidity sensor to monitor changes. This is to ensure that the system temperature was always stable (0.05°C over at least 45 mins of experimental duration). For temperature monitoring, after evacuation, we closed valves 3 and 4 followed by opening valves 1 and 2 (shown in Fig. S3) and monitoring the heat convection and conduction in the system with the pressure gauge. Normally, the sample was placed inside the sample cell in less than 30 sec after opening the incubator and remained at least 45 min for the gas pressure to stabilize before the pressure decay test. After the pressure was stabilized (0.005 psi for an experimental pulse pressure of 200 psi), it was deemed that there was no appreciable additional flow due to temperature variation in the system, as indicated by the rebound of the pressure decay curve. After reaching a unitary gas condition and stable temperature in the GPT experiment, valves 2 and 4 were closed, and the reference cell was filled with the probing gas (mostly non-reactive helium) at 200 psi. Valve 2 was then opened to release the pressure in the reference cell into the void volume in the sample cell, and the pressure decay for both reference and sample cells were recorded over time.

### SI5. Experimental conditions

We performed leakage tests by measuring the pressure variation with non-porous solids, such as steel balls, as any leakage would cause pressure variations and, accordingly, errors in permeability measurements of tight porous samples (Heller et al., 2014). Before the data from porous samples were analyzed, the leakage pressure from the steel ball experiment was subtracted from the sample data to correct the modest (<5% of the pressure levels used for permeability analyses) leakage effect.

The need for a unitary gas environment (a single gas used in both reference and sample cells) is needed to successfully measure permeability via the GPT method. The relative movement of gas molecules in the mass transfer process is driven by the gas density gradient in the system. During gas transport, the pressure variance was recorded and used to obtain the permeability coefficient. However, when the gas in both cells is different, e.g., helium in the reference and air in the sample cells, the mathematical analysis requires a complicated correction accounting for the mean molar mass and the average gas dynamic viscosity of the gas mixture. In this study, we present the calculation with the viscosity of mixed gases for the GPT in the SI1. Since the mixed gas environment is not recommended, air evacuation should be used for a well-controlled unitary gas environment in the GPT.

A stable temperature is another critical point to ensure the success of the GPT experiment. A sensitive pressure transducer in combination with the ideal gas law, used to establish the relationship between pressure and gas volume change, would be a much more convenient and precise way than the gas flow meter to determine the gas permeability considering the measurement accuracy. According to Amonton's law (Gao et al., 2004), the kinetic energy of gas molecules is determined by the temperature, and any changes would alter the molecular collision force causing a pressure variation and a volumetric error. The GPT experiments were run two or three times on the same sample, and the sample skeletal density at the end of the experiment were obtained to check the overall indication of leakage and temperature control. The experimental data with relatively large and stable skeletal density (mostly the last run, from small but appreciable pressure change to reach stable values) were used.

References:

Abdassah, D., Ershaghi, I., 1986. Triple-porosity systems for representing naturally fractured reservoirs. SPE Formation Evaluation 1(02), 113-127.

Barrer, R.M., 1941. Diffusion in and through solids, Рипол Классик.

Bibby, R., 1981. Mass transport of solutes in dual‐porosity media. Water Resour. Res. 17(4), 1075-1081.

Bock, H., Dehandschutter, B., Martin, C.D., Mazurek, M., De Haller, A., Skoczylas, F., Davy, C., 2010. Self-sealing of fractures in argillaceous formations in the context of geological disposal of radioactive waste.

Brace, W.F., Walsh, J.B., Frangos, W.T., 1968. Permeability of granite under high pressure. Journal of Geophysical Research 73(6), 2225-2236.

Brokaw, R.S., 1968. Viscosity of gas mixtures, National Aeronautics and Space Administration.

Busch, A., Alles, S., Gensterblum, Y., Prinz, D., Dewhurst, D.N., Raven, M.D., Stanjek, H., Krooss, B.M., 2008. Carbon dioxide storage potential of shales. Int. J. Greenh. Gas Control 2(3), 297-308.

Carslaw, H.S., Jaeger, J.C., 1959. Conduction of heat in solids.

Chen, X., Pfender, E., 1983. Effect of the Knudsen number on heat transfer to a particle immersed into a thermal plasma. Plasma Chem. Plasma Process. 3(1), 97-113.

Civan, F., Devegowda, D., Sigal, R.F., 2013. Critical evaluation and improvement of methods for determination of matrix permeability of shale, Society of Petroleum Engineers.

Cui, X., Bustin, A., Bustin, R.M., 2009. Measurements of gas permeability and diffusivity of tight reservoir rocks: different approaches and their applications. Geofluids 9(3), 208-223.

Devegowda, D., 2015. Comparison of shale permeability to gas determined by pressure-pulse transmission testing of core plugs and crushed samples, Unconventional Resources Technology Conference (URTEC).

Egermann, P., Lenormand, R., Longeron, D.G., Zarcone, C., 2005. A fast and direct method of permeability measurements on drill cuttings. Spe Reserv. Eval. Eng. 8(04), 269-275.

Fakher, S., Abdelaal, H., Elgahawy, Y., El-Tonbary, A., 2020. A Review of Long-Term Carbon Dioxide Storage in Shale Reservoirs, OnePetro.

Gao, J., Luedtke, W.D., Gourdon, D., Ruths, M., Israelachvili, J.N., Landman, U., 2004. Frictional forces and Amontons' law: from the molecular to the macroscopic scale, ACS Publications.

Gensterblum, Y., Ghanizadeh, A., Cuss, R.J., Amann-Hildenbrand, A., Krooss, B.M., Clarkson, C.R., Harrington, J.F., Zoback, M.D., 2015. Gas transport and storage capacity in shale gas reservoirs – A review. Part A: Transport processes. Journal of Unconventional Oil and Gas Resources 12, 87-122.

Ghanbarian, B., 2022a. Estimating the scale dependence of permeability at pore and core scales: Incorporating effects of porosity and finite size. Adv. Water Resour. 161, 104123.

Ghanbarian, B., 2022b. Scale dependence of tortuosity and diffusion: Finite-size scaling analysis. J. Contam. Hydrol. 245, 103953.

Ghanbarian, B., Hunt, A.G., Daigle, H., 2016. Fluid flow in porous media with rough pore‐solid interface. Water Resour. Res. 52(3), 2045-2058.

Guidry, K., Luffel, D., Curtis, J., 1996. Development of laboratory and petrophysical techniques for evaluating shale reservoirs. Final technical report, October 1986-September 1993, ResTech Houston, Inc., TX (United States).

Gutierrez, M., Øino, L.E., Nygaard, R., 2000. Stress-dependent permeability of a de-mineralised fracture in shale. Mar. Pet. Geol. 17(8), 895-907.

Haggerty, R., Gorelick, S.M., 1995. Multiple‐rate mass transfer for modeling diffusion and surface reactions in media with pore‐scale heterogeneity. Water Resour. Res. 31(10), 2383-2400.

Haskett, S.E., Narahara, G.M., Holditch, S.A., 1988. A method for simultaneous determination of permeability and porosity in low-permeability cores. SPE formation evaluation 3(03), 651-658.

Haszeldine, S., Lu, J., Wilkinson, M., Macleod, G., 2006. Long-timescale interaction of CO2 storage with reservoir and seal: Miller and Brae natural analogue fields North Sea. Greenhouse Gas Control Technology 8.

Heller, R., Vermylen, J., Zoback, M., 2014. Experimental investigation of matrix permeability of gas shalesExperimental Investigation of Matrix Permeability of Gas Shales. Aapg Bull. 98(5), 975-995.

Hu, Q., Ewing, R.P., Dultz, S., 2012. Low pore connectivity in natural rock. J. Contam. Hydrol. 133, 76-83.

Hu, Q., Ewing, R.P., Rowe, H.D., 2015. Low nanopore connectivity limits gas production in Barnett formation. Journal of Geophysical Research: Solid Earth 120(12), 8073-8087.

Huenges, E., 2016., pp. 743-761, Elsevier.

Javadpour, F., 2009. Nanopores and apparent permeability of gas flow in mudrocks (shales and siltstone). Journal of Canadian Petroleum Technology 48(08), 16-21.

Jia, B., Tsau, J., Barati, R., Zhang, F., 2019. Impact of Heterogeneity on the Transient Gas Flow Process in Tight Rock. Energies 12(18), 3559.

Kamath, J., 1992. Evaluation of accuracy of estimating air permeability from mercury-injection data. SPE Formation evaluation 7(04), 304-310.

Khosrokhavar, R., 2016. Shale gas formations and their potential for carbon storage: opportunities and outlook. Mechanisms for CO2 Sequestration in Geological Formations and Enhanced Gas Recovery, 67-86.

Kim, C., Jang, H., Lee, J., 2015. Experimental investigation on the characteristics of gas diffusion in shale gas reservoir using porosity and permeability of nanopore scale. J. Pet. Sci. Eng. 133, 226-237.

Kim, J., Kwon, S., Sanchez, M., Cho, G., 2011. Geological storage of high level nuclear waste. Ksce J. Civ. Eng. 15(4), 721-737.

Liu, W., Li, Y., Yang, C., Daemen, J.J., Yang, Y., Zhang, G., 2015. Permeability characteristics of mudstone cap rock and interlayers in bedded salt formations and tightness assessment for underground gas storage caverns. Eng. Geol. 193, 212-223.

Lu, J., Wilkinson, M., Haszeldine, R.S., Fallick, A.E., 2009. Long-term performance of a mudrock seal in natural CO2 storage. Geology 37(1), 35-38.

Luffel, D.L., Hopkins, C.W., Schettler Jr, P.D., 1993. Matrix permeability measurement of gas productive shales, Society of Petroleum Engineers.

Neuzil, C.E., 1986. Groundwater flow in low‐permeability environments. Water Resour. Res. 22(8), 1163-1195.

Neuzil, C.E., 2013. Can shale safely host US nuclear waste? Eos, Transactions American Geophysical Union 94(30), 261-262.

Peng, S., Loucks, B., 2016. Permeability measurements in mudrocks using gas-expansion methods on plug and crushed-rock samples. Mar. Pet. Geol. 73, 299-310.

Pini, R., 2014. Assessing the adsorption properties of mudrocks for CO2 sequestration. Energy Procedia 63, 5556-5561.

Profice, S., Lasseux, D., Jannot, Y., Jebara, N., Hamon, G., 2012. Permeability, porosity and klinkenberg coefficien determination on crushed porous media. Petrophysics 53(06), 430-438.

Rasmuson, A., 1985. The effect of particles of variable size, shape and properties on the dynamics of fixed beds. Chem. Eng. Sci. 40(4), 621-629.

Roy, S., Raju, R., Chuang, H.F., Cruden, B.A., Meyyappan, M., 2003. Modeling gas flow through microchannels and nanopores. J. Appl. Phys. 93(8), 4870-4879.

Ruthven, D.M., 1984. Principles of adsorption and adsorption processes, John Wiley & Sons.

Ruthven, D.M., Loughlin, K.F., 1971. The effect of crystallite shape and size distribution on diffusion measurements in molecular sieves. Chem. Eng. Sci. 26(5), 577-584.

Sutherland, W., 1895. XXXVII. The viscosity of mixed gases. The London, Edinburgh, and Dublin Philosophical Magazine and Journal of Science 40(246), 421-431.

Tarkowski, R., 2019. Underground hydrogen storage: Characteristics and prospects. Renewable and Sustainable Energy Reviews 105, 86-94.

Wu, T., Zhang, D., Li, X., 2020. A radial differential pressure decay method with micro-plug samples for determining the apparent permeability of shale matrix. J. Nat. Gas Sci. Eng. 74, 103126.

Yang, M., Annable, M.D., Jawitz, J.W., 2015. Back diffusion from thin low permeability zones. Environ. Sci. Technol. 49(1), 415-422.

Zhang, J.J., Liu, H., Boudjatit, M., 2020. Matrix permeability measurement from fractured unconventional source-rock samples: Method and application. J. Contam. Hydrol., 103663.

Zhang, T., Hu, Q., Chen, W., Gao, Y., Feng, X., Wang, G., 2021. Analyses of True-Triaxial Hydraulic Fracturing of Granite Samples for an Enhanced Geothermal System. Lithosphere 2021(Special 5).

Zhang, T., Zhou, W., Hu, Q., Xu, H., Zhao, J., Zhang, C., 2021. A pulse-decay method for low permeability analyses of granular porous media: Mathematical solutions and experimental methodologies.