Supporting Information

for

An updated microphysical model for particle activation in contrails: the role of volatile plume particles

Joel Ponsonby1, Roger Teoh1, Bernd Kärcher2, and Marc E. J. Stettler1*

1Department of Civil and Environmental Engineering, Imperial College London, London, SW7 2AZ, United Kingdom

2Institute for Atmospheric Physics, DLR Oberpfaffenhofen, Weßling, Germany

*Correspondence to: Marc E. J. Stettler (m.stettler@imperial.ac.uk)

Contents

S1 Homogeneous ice nucleation and 𝜅-Köhler theory	2

S2 Radial growth equation	4

S3 Limiting behaviour of 𝜅-Köhler theory	5

S4 Extension to lognormal distributions	6

S5 K15 model extension: particle modes	7

S6 K15 model extension: activated fractions	8

S7 K15 model extension: numerical treatment of κ-Köhler theory	9

S8 K15 model extension: microphysical term	10

S9 Model sensitivity to nvPM particle size	14

S10 Upper tropospheric ambient temperatures	16

S11 Binned pyrcel output	19

References	20

# S1 Homogeneous ice nucleation and 𝜅-Köhler theory

To estimate the homogeneous ice nucleation temperature for supercooled aqueous solution droplets, we employ a similar approach to the original K15 model. This method provides an estimate for the homogeneous temperature of pure water droplets i.e., those forming from wholly insoluble particles κ=0. Here, we extend this analysis to include particles that are not wholly insoluble (κ≠0), where the activity of the surrounding solution depends implicitly on the ambient conditions through the wet particle diameter, dw = dw(Sv, T) and explicitly on the particle properties, dd and κ, as described in Sect. 2.2 of the main text. Regardless, homogeneous ice nucleation is governed by the central expression

j = J ∙LWV,					  	 		                                                          (S1)

where j is the freezing rate (ice nucleation events per second), which is given as a product of the freezing rate coefficient J (ice nucleation events per unit volume per unit time) and the liquid water volume, LWV = 43π(r3- rd3). Assuming that a droplet entirely freezes when an ice nucleation event occurs within its volume, j is constrained as the number of droplets freezing per unit time. Previous studies have demonstrated that for typical cloud droplets with diameters of ~μm and homogeneous ice nucleation temperatures of ~235 K, ln(J) scales inversely proportional to temperature (Murray et al., 2010). If we consider a number of identical droplets within a cooling contrail, while plume temperatures are above the common homogeneous ice nucleation temperature (T > Thom), although LWV is maximised, j is limited by small J. As the contrail cools and transitions towards Thom, high plume cooling rates (< -10 K/s) lead to a sharp increase in J, maximising j. During the ice nucleation event, LWV is quickly depleted, and j returns to 0. Incidentally, if the droplet size distribution is monomodal, it is typically narrow enough to conform to the same narrative. Based on this semi-quantitative analysis, it is appropriate to assume a pulse-like nature of j and characterise the freezing event by defining a characteristic freezing timescale,

τfrz-1=∂lnj∂t≈∂lnJ∂TdTdt,			       		   		                                           	               (S2)

where we have assumed that changes in ln(LWV) are much slower than changes in ln(J). Using Eq. (S2), we can transform Eq. (S1) to estimate the frozen droplet fraction λ (Murray et al., 2012).

λ = 1-e-JτfrzLWV.			       						                             (S3)

For the below analysis, we choose λ = 1 in line with previous studies (Kärcher et al., 2015; Lewellen, 2020) and evaluate τfrz-1 according to Eq. (S2). We use the freezing rate coefficient described by (Koop et al., 2000), which depends inter alia on the activity of the solution, J=Jaw. For a given particle type defined by dd and κ and at a given temperature, T, first we evaluate dw and Sv for saturation ratios Sv<Sv,c. Next, we evaluate J=J(Sv, T) according to the relations outlined in Koop et al., (Koop et al., 2000) using an internal droplet pressure (P) parameterization (Marcolli, 2020). This enables us to evaluate P = P (Sv, T) for saturation ratios Sv<Sv,c. We can then finally determine the critical saturation ratio, Sv,hom at which we satisfy our predefined threshold, λ = 1. This critical saturation ratio represents approximate conditions for homogeneous ice nucleation within the given particle mode.

Figure S1: illustrative ice nucleation temperature as predicted using J parametrizations from (Koop et al., 2000) using the methodology outlined in (Kärcher et al., 2015). Particle properties have been taken from Table. 1 in the main text, assuming a sulphur-rich vPM mode with GMDvPM = 4 nm. Several contrail mixing lines have also been presented with G = 1.64.

In Fig. S1, we show the critical saturation ratios required from particle activation and homogeneous ice nucleation, Sv,c and Sv,hom respectively, for the three particle modes outlined in Table. 1 of the main text for a sulphur-rich vPM mode. The paths traced by Sv,hom=Sv,hom(T) are also commonly known as “Koop line” freezing thresholds as described by (Koop et al., 2000) and in other literature (Baumgartner et al., 2022). For a given particle type, if the ambient conditions lie in the region above the relevant Koop line, the solution droplets are able to freeze homogeneously. The Koop line termini are indicated using coloured markers on Fig. S1, these identify the temperatures beyond which the condition on λ is no longer achieved; for temperatures below the termini, Sv,hom asymptotically approaches Sv,c.We have also shown three contrail mixing lines at three different ambient temperatures corresponding to ∆TSAC of 0 K, 5 K and 10 K. Fig. S1 enables us to determine, for a given particle type, whether particle activation or homogeneous ice nucleation is likely to occur first during the contrail mixing process. We find that for the nvPM and vPM modes (brown and blue lines, respectively), every feasible contrail mixing line first surpasses the critical saturation ratio for droplet activation (bisects Sv,c) before entering the region above the relevant Koop line. For that reason, we conclude that for the majority of vPM and nvPM particles, droplet activation takes place prior to homogeneous ice nucleation. The same conclusion is met for the hygroscopic ambient mode, however for small ∆TSAC ~ 0.1 K, it is possible that a small fraction of the larger particles may freeze homogeneously before they are considered activated. In conclusion, we find that for the three types of particles investigated within this work, we are justified in assuming that particle activation occurs prior to homogeneous ice nucleation.

# S2 Radial growth equation

The growth or evaporation of droplets by diffusion of water vapour is described by the radial growth law which may be expressed in the general form (Seinfeld and Pandis, 2016)

drdt=GrSe-Sv ,						   					               (S4)

where Se is the ambient (environmental) saturation ratio, Sv is the equilibrium water saturation ratio above the particle (see Sect. 2.2 of the main text) and G is a growth coefficient. The growth coefficient may be expressed as the sum of two terms, the first of which is associated with heat conduction and the second is associated with water vapour diffusivity (Rogers and Yau, 1996)

G=1Fk+Fd; Fk=lwρw4ka'TlwMwRT-1,  Fd=ρwRT4es0Dw'Mw,					 	                             (S5)

where lw is the latent heat of evaporation of water. Parameters Dw' and ka' represent the modified diffusivity (Dw) and thermal conductivity (ka), respectively, accounting for non-continuum effects as (Pruppacher and Klett, 2010)

ka'=ka1+kaαTrρwcp2πMaRT; ka=10-34.39+0.071T, 				                      	               (S6)

Dw'=Dw1+DwαCr2πMaRT; Dw=2.11×10-5T273.151.94P0PT,	                                                                        (S7)

where we have introduced the heat capacity cp and the thermal accommodation and condensation coefficients, αC and αT respectively (P0 = 101325 Pa). And, for the purposes of this work, we assume fixed coefficients αT=αC=1. If the effects of diffusivity are much greater than thermal conductivity (i.e., Fd>>Fk), we can simplify Eq. (S4) and rearrange it into the equivalent form

drdt=Ab2rw + 1;A =υνnwsatαw4Se-Sv, b2=ν4D, 			                                    			(S8)

where ν is the molecular thermal speed. This definition is employed when solving the Lw integral within the K15 approach (Ford, 1998, p.20076; Kärcher et al., 2006; Kärcher and Lohmann, 2003) as the functional dependence on rw makes Eq. (S8) more tractable than Eq. (S4).

# S3 Limiting behaviour of 𝜅-Köhler theory

We know that the maximum hygroscopicity parameter is on the order 𝜅 ~ 1. Using this value, Eq. (7) in the main text collapses to

Sv=dw3-dd3dw3e4σsMwRTρwdw.						    	                 		               (S9)

To identify the critical wet particle diameter, dw, c, we can differentiate Eq. (S9) with respect to dw and set the result equal to zero.

dSvddw=3ⅇadwdw-ⅇaw-dd3+dw3dw5-3ⅇaw-dd3+dw3dw4=0;   a=4σsMwRTρw,		      	     	                                          (S10)

dw, c=213dd3dd3a3+-4dd9a3+dd6a613+dd3a3+-4dd9a3+dd6a613213a.		      	     	     	                           (S11)

To recover the critical water saturation ratio, we can introduce the result from Eq. (S11) into Eq. (S9), which gives us an analytic expression for Sv,  c=Sv,  ca, dd, κ=1. We find that this analytical function is monotonic, and the solution asymptotically approaches Sv,  c→1 in the limit dp→∞, in agreement with numerical solution used in this work. Therefore, we have demonstrated that within the κ-Köhler framework, even highly hygroscopic materials (κ~1) cannot achieve critical water saturation ratios that are equivalent to or fall below unity. Only surfaces with zero curvature (plane surfaces) are capable of condensing water at ambient water saturation ratios of unity; for particles with finite diameters, the equality Sv,  c>1, is always satisfied.

# S4 Extension to lognormal distributions

In the analysis in Sect. 2 of the main text, we have presented arguments under the assumption that all particles within a contrail mixing plume are identical and therefore share the common properties (dd, κ), which define their activation behaviour. Although this approach is sufficient to gain an insight into the relevance of particle properties, most of the results presented hereafter in this work rely on – and can only be interpreted by - a more sophisticated description of particle properties. To that end, we define a particle type according to its particle size distribution (PSD) (Seinfeld and Pandis, 2016)

dNdlndd=Nt2πlnσge-lndd-lndd22lnσg2,		            					                                          (S12)

where Nt represents the total number of particles, σg represents the geometric standard deviation (GSD) of the distribution and dd represents the geometric mean diameter (GMD). Incidentally, the description used in Sect. 2 of the main text is equivalent to taking Eq. (S12) in the limit σg→ 1, which represents an infinitely narrow distribution comprising particles with a uniform diameter dd=dd. Assuming that a given particle type comprises particles with uniform composition and mixing state, we assert that a particle type may be fully described if the properties of its PSD (dd,σg, Nt) and its total hygroscopicity parameter are known. A consequence of departing from the simplistic description of an infinitely narrow PSD, is that for a given ambient saturation ratio, it is no longer the case that either all or none of the particles are activated (Boolean-type), but rather that some proportion of the particles will be activated. From Sect. 2 of the main text, we know that the critical saturation ratio for activation may be determined if the particle diameter, total hygroscopicity parameter and ambient temperature are known, Sv,  c=Sv,  cT, dd, κ. Similarly, this relationship may be inverted to estimate the critical particle diameter, dd,  c, required for activation for a given ambient saturation ratio, dd,  c=dd,  cT, Sv, κ. Because the relationship is monotonic, we know that for a given ambient saturation ratio Sv, all particles with dd>dd,  c will be able to activate. Therefore, for a particle type defined by a total hygroscopicity parameter κ and PSD properties (dd,σg, Nt), the number of particles that can activate at a given ambient water saturation ratio, Sv, is given by the truncated integral of Eq. (S12) as

N=dd,  cT, Sv, κ∞Nt2πlnσge-lndd-lndd22lnσg2dlndd.		      	      		   	                           (S13)

After changing variables and performing the integration, we arrive at a modified form of the lognormal cumulative distribution function which uses the error function (erf)

N=Nt21-erflndd,  cT, Sv, κ-lndd22lnσg2.		         			   	 	                           (S14)

# S5 K15 model extension: particle modes

To extend the description of the ambient particle mode used in K15, results from the global aerosol-climate model ECHAM-HAM (Zhang et al., 2012) were obtained. This model categorises particles into seven classes according to their size and solubility (Vignati et al., 2004): insoluble coarse mode particles and insoluble/soluble nucleation, accumulation and Aitken mode particles. Each class is described by a GMD and GSD and, as an approximation, soluble and insoluble modes are prescribed conservative hygroscopicity parameters of 0.5 and 0 respectively. For the purposes of this work, ECHAM-HAM data for year 2018 were interpolated at the mean altitude of waypoints that satisfied the SAC (Spire Aviation, 2024).

Figure S2: (a) anticipated activated particle number concentration for the original and updated ambient particle modes. Activation of the small, soluble nucleation mode is highlighted, resulting in a pronounced increase in the number concentration of activated particles in the plume (b) comparison of the original and updated ambient particle modes used within the K15 and extended K15 models respectively.

In Fig. S2b, we compare the ambient particle size distribution obtained from the 7-mode ECHAM-HAM model (blue) with the single ambient mode used in the K15 model (beige). The resulting differences in particle activation are shown in Fig. S2a where the total number of activated particles is shown as a function of ambient supersaturation. We find that for moderate supersaturations (sv < 10 %) both descriptions lead to similar numbers of activated particles. However, if the supersaturation is able to exceed ~ 30 %, the populous soluble nucleation mode is able to activate resulting in a sharp increase in the number of activated particles. Under soot-poor conditions, the absence of a significant condensational sink will typically lead to greater plume supersaturations. Hence, a complete description of the ambient particle mode (including soluble nucleation mode particles) is critical to understanding contrail formation under these conditions (Bier et al., 2022).

# S6 K15 model extension: activated fractions

In the K15 model, integration over the PDF to determine activated number concentrations is approximated. The form of the approximation used is given by

N≈Nt1+dd,  cT, Sv, κddζ,		      	     				        		                           (S15)

ζ=42πlnσg,		      	     				   	        	    	                           (S16)

where ζ is defined as a size distribution slope parameter.

Figure S3: Comparison of the K15 approximation of PDF integration versus full (analytical) solution for the three particle modes as outlined in Table. 1 of the main text.

Fig. S3 shows the agreement between Eq. (S15) and Eq. (S16). Using the particle types outlined in Table. 1 of the main text, we find that for nvPM and ambient particle modes, the K15 approximation is valid for estimates above a supersaturation of ~ 10%. Below this value, the K15 approximation systematically overpredicts activated number concentrations. For the transition and soot-poor regimes, discrepancy between the K15 approximation and the full solution is more prominent, and agreement is only achieved for supersaturations exceeding ~ 20%. For this reason, we choose to use the full solution in our extended K15 model simulations.

# S7 K15 model extension: numerical treatment of κ-Köhler theory

In the K15 model, the authors estimate Sv,  cT, dd, κ using an approximate analytical solution derived from Eq. (7) in the main text, which takes the form

lnSv, c=rkdd354κ; rk=4σsMwRTρw,								                           (S17)

where the Kelvin diameter, dk, is defined according to the exponent (dd/rk) in the Kelvin equation. Although analytically tractable, Eq. (S17) is only strictly valid for estimating critical water saturation ratios provided κ > 0.2 (Petters and Kreidenweis, 2007). Outside this range,  Sv,  cT, dd, κ must instead be determined by numerical methods.

Figure S4: Comparison of numerical versus analytical treatment of κ-Köhler theory at T = 230 K.

In Fig. S4, we compare the analytical and numerical approaches to estimating Sv,  c. Estimates have been shown for two dry particle diameters, dd = 5 nm and 50 nm, to highlight the size dependence. In summary, we find that the condition κ > 0.2 is valid (i.e., the numerical and analytical approximation agree to within ~ 1%) provided dd ≳ 10 nm. The approximation collapses for smaller dry particle diameters and overpredicts the numerical result by a factor ~1.3 and ~10 for dd= 5 nm and dd= 1 nm (not shown) respectively (at κ = 0.2). Therefore, for the purposes of this work, the analytical model is strictly applicable for particles that satisfy both dd > 10 nm and κ > 0.2. Whilst these conditions are met for the majority of the nvPM and ambient particles considered in K15, they are not met for smaller vPM particles. Therefore, we choose to apply the full numerical solution to evaluate Sv, c in our extended K15 model simulations, at the cost of a marginal decrease in computational efficiency.

# S8 K15 model extension: microphysical term

To determine nw(2)t, first we need to identify the temporal dependence of Lw. This is achieved by approximating the functional dependence of nwt0 as (Ford, 1998)

nwt0=nw(t)e- t- t0τact,	     	                                  	                                                                                     (S18)

where τact is the characteristic timescale of activation within the mixing plume. This prescribes that the largest contribution to Lwt occurs from droplet activation and growth within a narrow activation window (t – t0) ~ τact, in a burst-like activation event. For additional justification on the form of the ansatz, the reader is directed to the original texts (Ford, 1998; Kärcher and Lohmann, 2003). Note that Eq. (S18) allows us to define the total number of activated particles in terms of the nucleation rate, nw(t)=nwtτact. Hence, using this relation along with Eq. (S18), we can transform Eq. (S18) as

Lw  =4πnwsatυnw(t)τact0∞dr0dnwdr0-∞te- t- t0τactrw2t, t0rw(t, t0) dt0,  		                         	                                          (S19)

where we have used the ideal gas law to redefine the prefactor in terms of nwsat and υ, the water vapor number density at water saturation and the molar volume of water respectively. The definition of the radial growth law, rw, is described in more detail in SI S2. Critically, our definition of the radial growth law departs from the definition used in K15 and earlier work (Ford, 1998; Kärcher and Lohmann, 2003), as we have chosen to explicitly incorporate the Kelvin effect. This enables us to more accurately describe water uptake by the smaller vPM mode. For efficient computational models, it is preferable that the solutions to Eq. (S19) are analytic, which is only possible if both integrations can be solved analytically. Therefore, we will next outline several approaches to solving the integrations.

The outer integration takes place over the activated particle size distribution. Although this can be solved numerically, there are also several approximations that can be used to yield compact analytical solutions. In the K15 model, the authors replace this integral using the critical droplet radius, weighted by the available number concentration of each particle type, rw, c. This approach assumes that all contributions made at time t0 are from particles with the critical radius rw, c. However, as we have shown in Sect. 2.2 of the main text, particles with radii r>rw, c are also able to activate. In other words, it is possible that this approach underestimates the mean activated particle radius at a given time t0. We explore a replacement for the first integral by using the arithmetic mean particle radius over the truncated activated size distribution, (r>rw, c), which is then weighted by the available number concentration of each particle type. We will later show under which circumstances this approach provides better agreement with the numerical solution than the original K15 model approach.

Within the K15 model, the authors perform the inner integration by separating the integral into two cases (Kärcher and Lohmann, 2003). To explain the physical meaning of these cases, we first define the relaxation parameter, κw, as the ratio of the droplet activation and growth timescales, τact and τgw respectively,

κw=τactτgw;  τgw=1∂lnrw∂t=rw(1+ b2rw)A,			                                      		                           (S20)

where the prefactor, A can either include or omit the Kelvin effect and is described in SI S2. This enables us to define the first case as the slow-growth regime, where particle activation dominates over droplet growth and κw<1. The second case

is the fast-growth regime, where droplet growth dominates over particle activation and κw>1. These cases permit us to prescribe limiting behaviour in the droplet growth law, thereby eliminating the t-dependence in Eq. (S19). For a full description of this method, the reader is directed to (Kärcher and Lohmann, 2003). When the Kelvin effect is not included in the growth law, analytical solutions for the inner integration can be determined in both the slow- and fast-growth regimes. On the other hand, incorporating the Kelvin effect within the radial growth law precludes an analytical approach, and the resulting expression can only be solved numerically. However, the slow-growth regime can still be solved numerically, and the analytic solution is given by

Lw  =4πrw,c3nw(t) nwsatτgwυ2κw2+2κw+1,			                                 			                           (S21)

including our earlier approximating regarding the outer integration. If the Kelvin effect is omitted, it is also possible to solve Eq. (S19) analytically for all growth regimes as described in Kärcher et al., (Kärcher et al., 2006).

Figure S5: variation of nw(2) and nw(1) with increasing plume supersaturation for TA = 215 K and G = 1.64 PaK-1 in the soot-rich regime (EInvPM = 1015 kg-1), using the particle properties GMD, GSD and 𝜅 defined in Table of the main text. In panels (a) and (b), EIvPM = 0 kg-1 (as in the original K15 model) whereas in panels (c) and (d), EIvPM = 1017 kg-1. In panels (a) and (c) nw(2) has been computed for the slow-growth regime with inclusion (inc.) and exclusion (exc.) of the Kelvin effect. For each of these, the outer integral has been estimated using three approaches: solving numerically (thinner purple lines), using the B15 model (green lines) and using our extension (brown lines). In panels (b) and (d) nw(2) has been computed for the fast-growth regime with inclusion (inc.) and exclusion (exc.) of the Kelvin effect. Also, results valid across all regimes have been plotted (thicker purple lines), excluding the Kelvin effect (Kärcher et al., 2006). In each case, the outer integral has been estimated using our extension. In panel (a/c) and (b/d), vertical lines have also been shown corresponding to κw=0.1 and κw=10 respectively, defining the approximate limits of applicability according to Eq. (S20).

In Fig. S5, we have demonstrated how nw(1) and nw(2) vary with plume supersaturation under typical contrail-forming conditions (TA = 215 K, G = 1.64 PaK-1). These have been estimated in the soot-rich regime (EInvPM = 1015 kg-1) with and without vPM, using particle characteristics outlined in Table. 1 of the main text. In Fig. S5a, we show the variation of nw(2) deriving from the solutions for Lw  in the slow-growth regime. We find that within its range of validity (κw<0.1), incorporating the Kelvin effect can have a pronounced impact on estimates of nw, T for soot-rich conditions. In all cases, inclusion of the Kelvin effect acts to increase the effective number of contrail ice crystals, ni. This is because inclusion of the Kelvin effect suppresses the radial growth of water droplets, see Eq. (S8). Moreover, the size-dependence of the Kelvin effect means that if the available particles are smaller, the effect will be more pronounced. This is illustrated in Fig. S5c, where the relative difference between the solid (exc. Kelvin effect) and dashed (inc. Kelvin effect) lines increases dramatically once vPM activation becomes dominant (for sw > 30%) and rw, c approaches GMDvPM. We can also use Fig. S5a to compare the various approaches to evaluating the outer integral. Taking the numerical approach as a benchmark, we find better agreement using our approach than the original K15 approach, provided the Kelvin effect is included. This is due to the exponential dependence in Eq. (7) of the main text, which is highly sensitive to differences in the prediction of mean activated particle radius, rw, c-  rw, c. However, when the Kelvin effect is omitted, both approaches provide comparable results and here we choose to use the original K15 approach. Additionally, we find that when the vPM mode is included, it is no longer feasible to estimate nw(2) for the slow growth regime using the K15 approach with incorporation of the Kelvin effect. For this reason, there is no dashed green line shown in Fig. S5c.

In Fig. S5b and Fig. S5d, we show fast-growth regime solutions that include our outer integral extension with and without inclusion of the Kelvin effect. Inclusion of the Kelvin effect is relatively less impactful in the fast-growth regime solutions compared to the corresponding slow-growth solutions. This aligns with intuition, as when the microphysical term is dominated by droplet growth (as opposed to activation), the initial aerosol properties become less important (Kärcher et al., 2006). In Fig. S5b and Fig. S5d we also show the solution originally derived in Kärcher et al., 2006 that is valid across both regimes (albeit without inclusion of the Kelvin effect) and illustrate its consistency with the slow- and fast-growth solutions under their respective ranges of validity. Critically, when vPM particles are incorporated in the model – see Fig. S5c and Fig. S5d – the intersection of nw(1) and nw(2) occurs for κw>10 and therefore should only be described using solutions for the fast-growth regime. For this reason, we have chosen to adopt the result outlined by Kärcher et al., 2006, which is applicable across all growth regimes. In doing so, we are forced to omit the Kelvin effect and our updated approach for estimating

# S9 Model sensitivity to nvPM particle size

The properties of aircraft nvPM particles can be investigated by performing ground-based measurements behind turbofan engines (Saffaripour et al., 2020). Primary particle size distribution characteristics are typically determined offline, using samples collected for Transmission Electron Microscopy (TEM). Image analysis software is then used to extract information from individual samples, including aggregate scaling properties, the geometric mean primary particle diameter (GMDnvPM, p) and its standard deviation (GSDnvPM, p) (Sipkens et al., 2023). Using Table. 2 in (Saffaripour et al., 2020), we have selected and/or extracted relevant primary particle size distribution characteristics for samples collected at 100% thrust from turbofan engines burning Jet A-1, which were obtained from TEM images. To that end, using data from (a) (Liati et al., 2014) GMDnvPM, p = 17.4 nm, GSDnvPM, p = 1.54; (b) (Marhaba et al., 2019) GMDnvPM, p = 13.5 nm, GSDnvPM, p = 1.50 and (c) (Delhaye et al., 2017) GMDnvPM, p = 16.1 nm, GSDnvPM, p = 1.50, we estimate average primary particle size distribution characteristics GMDnvPM, p = 15.7 ± 1.6 nm and GSDnvPM, p = 1.50 ± 0.02. Given our choice of aggregate particle diameter (GMDnvPM = 35 nm), the ratio of mean primary-to-aggregate particle sizes is XR = 0.45, which is similar to the baseline value XR = 0.41 used in (Yu et al., 2024). However, hereafter we choose to discriminate between primary or aggregate particle size distribution characteristics using the subscript p/a as both the GMD and the GSD are modified.

Figure S6: apparent ice emission index at 230 K (hexagonal markers) and its dependence on TA and EInvPM for (a) sulphur-rich and (b) organic-rich volatile particles. The impact of assuming aggregate “(a)” or primary “(p)” particle size distribution characteristics, GMD and GSD of (35 nm, 2.0) and (15.7, 1.5) respectively, is shown.

Fig. SX shows AEIice sensitivity to TA, EInvPM and the choice of either nvPM primary particles or aggregates, using (a) sulphur-rich and (b) organic-rich vPM properties as described in Sect. 3.4 of the main text. For temperatures close to TSAC, AEIice is reduced if most droplets are formed via nvPM. This is because for a given plume supersaturation, more nvPM aggregate particles are able to activate than nvPM primary particles, due to the larger GMD and GSD. Therefore, when EInvPM = 1016 – which precludes substantial vPM activation in the case of both sulphur- and organic-rich vPM - AEIice is reduced by 30% when using nvPM primary particles compared to aggregates. For sulphur-rich conditions, as the minimum is approached from high EInvPM which implies competition between vPM and nvPM, AEIice is increased by up to 40% when using nvPM primary particles compared to aggregates. Again, this occurs because fewer nvPM primary particles are able to activate at a given supersaturation compared to aggregates, so the more numerous vPM mode is able to compete for available water vapour at a higher EInvPM.

Overall, we find that upon changing our description from nvPM aggregates to primary particles: (i) if most water droplets form via nvPM, AEIice is reduced as a smaller proportion of the nvPM size distribution can activate and (ii) if a large proportion of droplets form via vPM (i.e., the minimum in Fig. SX), AEIice is increased as nvPM provides less competition for available supersaturation and (iii) for sulphur-rich vPM, (i) + (ii) lead to an effective translation of the aggregate profiles in Fig. SX (a) towards increased EInvPM. Each of these results is qualitatively consistent with the findings in (Yu et al., 2024).

# S10 Upper tropospheric ambient temperatures

Meteorological and  Automatic Dependent Surveillance-Broadcast (ADS-B) data were obtained from Spire Aviation (Spire Aviation, 2024) and ECMWF respectively for the date 01/01/2019. The purpose of this analysis is to assess the upper tropospheric lapse rate at typical contrail-forming altitudes and comment on implications for temperature-based contrail avoidance.

Figure S7: upper tropospheric lapse rate (ΔT/Δz) at the mean contrail-forming altitude obtained for flight waypoints satisfying the SAC using the Spire and ECMWF datasets for 01/01/2019.

Fig. S7 shows how the upper tropospheric lapse rate (ΔT/Δz) calculated at the mean contrail-forming altitude (contrail waypoints that satisfy the SAC) varies as a function of longitude and latitude. We find that within the tropics (± 23.5°), the lapse rate is approximately – (8 ± 1) K/km; the temperature increases with a decrease in altitude. Here, ambient temperature changes of ~2 K are possible by reducing cruise altitudes by ± 10 flight levels (1000 ft). By contrast, in the extratropical regions, the lapse rate exhibits greater variation and can be positive.

Figure S8: attenuation in AEIice achieved for soot-poor conditions (EInvPM = 1012 kg-1) with deviations of ± 10 flight levels from the mean contrail-forming altitude.

In Fig. 9 of the main text, we presented the dependence of AEIice on ambient temperature for various emissions scenarios. Using these data with the information in Fig. S7, we can estimate the reduction in AEIice (without sublimation losses) that could be achieved by altering flight altitudes by ± 10 flight levels. For the purpose of this discussion, we have focussed on the soot-poor regime, where the greatest attenuation of the apparent emission index of ice (AEIice, unadjusted / AEIice, with altitude adjustment) is possible (see Fig. 9 of the main text). We find that attenuations exceeding 100 are achievable in the tropics by reducing flight altitudes by 10 flight levels. Smaller attenuations are achievable in the extratropical regions as the ambient temperature at the mean contrail-forming altitude is cooler.

Figure S9: attenuation in AEIice as in Fig. SX. The value of log10(AEIice) at the mean contrail-forming altitude has also been marked on each grid cell for reference.

Figure S10: absolute reduction in AEIice achieved for soot-poor conditions (EInvPM = 1012 kg-1) with deviations of ± 10 flight levels from the mean contrail-forming altitude.

Although larger attenuations are achieved in the tropics compared to the extratropical regions, warmer ambient temperatures mean that absolute AEIice emissions at the mean contrail-forming altitude are typically lower, see Fig. S10. Absolute reductions in AEIice (AEIice, unadjusted - AEIice, with altitude adjustment) therefore provide a more suitable metric for targeting the most warming contrails, which are associated with larger AEIice (Teoh et al., 2022). To that end, we find that the largest absolute reductions in AEIice are found in the northern midlatitudes (~ 30° - 60°). This covers some of the densest regions of global air traffic, including Europe and North America (Teoh et al., 2023). Moreover, the above analysis was performed using meteorological data during winter in the Northern Hemisphere. During the summer months, upper tropospheric temperatures in the Northern Hemisphere are warmer so the potential for temperature-based mitigation in this region is improved. Overall, avoiding cooler regions of the atmosphere may provide an alternative strategy for contrail mitigation.

# S11 Binned pyrcel output

For a given aerosol mode, pyrcel tracks the size, concentration and phase (solid/liquid aerosol, liquid droplet or ice crystal) during plume evolution.

Figure S11: particle size distributions for (a) nvPM and (b) vPM illustrative of typical contrail mixing behaviour. The particle size distributions have been shown at four different temperatures during plume evolution and the phase (solid/liquid aerosol, liquid droplet, ice crystal) of each bin has been indicated using a different marker.

# References

Spire Aviation: https://spire.com/wiki/how-ads-b-has-shaped-the-modern-aviation-industry/, last access: 19 December 2024.

Baumgartner, M., Rolf, C., Grooß, J.-U., Schneider, J., Schorr, T., Möhler, O., Spichtinger, P., and Krämer, M.: New investigations on homogeneous ice nucleation: the effects of water activity and water saturation formulations, Atmospheric Chemistry and Physics, 22, 65–91, https://doi.org/10.5194/acp-22-65-2022, 2022.

Bier, A., Unterstrasser, S., and Vancassel, X.: Box model trajectory studies of contrail formation using a particle-based cloud microphysics scheme, Atmos. Chem. Phys., 22, 823–845, https://doi.org/10.5194/acp-22-823-2022, 2022.

Delhaye, D., Ouf, F.-X., Ferry, D., Ortega, I. K., Penanhoat, O., Peillon, S., Salm, F., Vancassel, X., Focsa, C., Irimiea, C., Harivel, N., Perez, B., Quinton, E., Yon, J., and Gaffie, D.: The MERMOSE project: Characterization of particulate matter emissions of a commercial aircraft engine, Journal of Aerosol Science, 105, 48–63, https://doi.org/10.1016/j.jaerosci.2016.11.018, 2017.

Ford, I.: Ice nucleation in jet aircraft exhaust plumes, in: Pollution from aircraft emissions in the North Atlantic flight corridor (POLINAT 2): final report, Office for Official Publ. of the European Communities, Luxembourg, 269–287, 1998.

Kärcher, B. and Lohmann, U.: A parameterization of cirrus cloud formation: Heterogeneous freezing, Journal of Geophysical Research: Atmospheres, 108, https://doi.org/10.1029/2002JD003220, 2003.

Kärcher, B., Hendricks, J., and Lohmann, U.: Physically based parameterization of cirrus cloud formation for use in global atmospheric models, Journal of Geophysical Research: Atmospheres, 111, https://doi.org/10.1029/2005JD006219, 2006.

Kärcher, B., Burkhardt, U., Bier, A., Bock, L., and Ford, I. J.: The microphysical pathway to contrail formation, JGR Atmospheres, 120, 7893–7927, https://doi.org/10.1002/2015JD023491, 2015.

Koop, T., Luo, B., Tsias, A., and Peter, T.: Water activity as the determinant for homogeneous ice nucleation in aqueous solutions, Nature, 406, 611–614, https://doi.org/10.1038/35020537, 2000.

Lewellen, D. C.: A Large-Eddy Simulation Study of Contrail Ice Number Formation, Journal of the Atmospheric Sciences, 77, 2585–2604, https://doi.org/10.1175/JAS-D-19-0322.1, 2020.

Liati, A., Brem, B. T., Durdina, L., Vögtli, M., Arroyo Rojas Dasilva, Y., Dimopoulos Eggenschwiler, P., and Wang, J.: Electron Microscopic Study of Soot Particulate Matter Emissions from Aircraft Turbine Engines, Environ. Sci. Technol., 48, 10975–10983, https://doi.org/10.1021/es501809b, 2014.

Marcolli, C.: Technical note: Fundamental aspects of ice nucleation via pore condensation and freezing including Laplace pressure and growth into macroscopic ice, Atmos. Chem. Phys., 20, 3209–3230, https://doi.org/10.5194/acp-20-3209-2020, 2020.

Marhaba, I., Ferry, D., Laffon, C., Regier, T. Z., Ouf, F.-X., and Parent, P.: Aircraft and MiniCAST soot at the nanoscale, Combustion and Flame, 204, 278–289, https://doi.org/10.1016/j.combustflame.2019.03.018, 2019.

Murray, B. J., Broadley, S. L., Wilson, T. W., Bull, S. J., Wills, R. H., Christenson, H. K., and Murray, E. J.: Kinetics of the homogeneous freezing of water, Phys. Chem. Chem. Phys., 12, 10380–10387, https://doi.org/10.1039/C003297B, 2010.

Murray, B. J., O’Sullivan, D., Atkinson, J. D., and Webb, M. E.: Ice nucleation by particles immersed in supercooled cloud droplets, Chem. Soc. Rev., 41, 6519, https://doi.org/10.1039/c2cs35200a, 2012.

Petters, M. D. and Kreidenweis, S. M.: A single parameter representation of hygroscopic growth and cloud condensation nucleus activity, Atmospheric Chemistry and Physics, 7, 1961–1971, https://doi.org/10.5194/acp-7-1961-2007, 2007.

Pruppacher, H. R. and Klett, J. D.: Microphysics of clouds and precipitation, 2nd ed., Springer Dordrecht, Dordrecht, 2010.

Rogers, R. R. and Yau, M. K.: A short course in cloud physics, Third edition., Butterworth-Heinemann, Burlington, Massachusetts, 1 pp., 1996.

Saffaripour, M., Thomson, K. A., Smallwood, G. J., and Lobo, P.: A review on the morphological properties of non-volatile particulate matter emissions from aircraft turbine engines, Journal of Aerosol Science, 139, 105467, https://doi.org/10.1016/j.jaerosci.2019.105467, 2020.

Seinfeld, J. H. and Pandis, S. N.: Atmospheric chemistry and physics: from air pollution to climate change, John Wiley & Sons, Inc, Hoboken, New Jersey, 2016.

Sipkens, T. A., Boies, A., Corbin, J. C., Chakrabarty, R. K., Olfert, J., and Rogak, S. N.: Overview of methods to characterize the mass, size, and morphology of soot, Journal of Aerosol Science, 173, 106211, https://doi.org/10.1016/j.jaerosci.2023.106211, 2023.

Teoh, R., Schumann, U., Gryspeerdt, E., Shapiro, M., Molloy, J., Koudis, G., Voigt, C., and Stettler, M. E. J.: Aviation contrail climate effects in the North Atlantic from 2016 to 2021, Atmos. Chem. Phys., 22, 10919–10935, https://doi.org/10.5194/acp-22-10919-2022, 2022.

Teoh, R., Engberg, Z., Shapiro, M., Dray, L., and Stettler, M.: A high-resolution Global Aviation emissions Inventory based on ADS-B (GAIA) for 2019–2021, https://doi.org/10.5194/egusphere-2023-724, 1 June 2023.

Vignati, E., Wilson, J., and Stier, P.: M7: An efficient size-resolved aerosol microphysics module for large-scale aerosol transport models, Journal of Geophysical Research: Atmospheres, 109, https://doi.org/10.1029/2003JD004485, 2004.

Yu, F., Kärcher, B., and Anderson, B. E.: Revisiting Contrail Ice Formation: Impact of Primary Soot Particle Sizes and Contribution of Volatile Particles, Environ. Sci. Technol., 58, 17650–17660, https://doi.org/10.1021/acs.est.4c04340, 2024.

Zhang, K., O’Donnell, D., Kazil, J., Stier, P., Kinne, S., Lohmann, U., Ferrachat, S., Croft, B., Quaas, J., Wan, H., Rast, S., and Feichter, J.: The global aerosol-climate model ECHAM-HAM, version 2: sensitivity to improvements in process representations, Atmospheric Chemistry and Physics, 12, 8911–8949, https://doi.org/10.5194/acp-12-8911-2012, 2012.