Supplement S2 of

Regional CO2 and CH4 inversion system using WRF-Chem (v4.4)/ DART (v9.8.0) and continuous high-precision observations over the Korean Peninsula

Doyoon Kwon et al.

Correspondence to: Jinkyu Hong (jhong@yonsei.ac.kr)

• Controlled twin observation system simulation experiment under perfect-model assumptions

• Figure S15 – S17

S1 Controlled twin observation system simulation experiment (OSSE) under perfect-model assumptions

This supplement provides the OSSE design and results as a controlled, perfect-model diagnostic of the WRF-Chem/DART state-augmentation machinery. It is intended to document algorithmic behavior and information-content sensitivity under known-truth conditions, not to demonstrate real-world robustness to transport-model error, boundary-condition bias, or representativeness error.

S2 Purpose, scope, and limitations

The controlled twin OSSE is provided as an auxiliary diagnostic of the WRF-Chem/DART state-augmentation machinery rather than as evidence of operational robustness. The OSSE serves two specific objectives: (i) to test whether known concentration innovations can be transferred into physically interpretable emission increments through ensemble-estimated concentration–flux covariances, and (ii) to compare the relative information content of the current 3-site network and a denser synthetic surface network under idealized perfect-model assumptions. The experiment is intentionally idealized: the nature run and assimilation experiment use the same WRF-Chem physics, transport operators, and boundary-condition treatment, and therefore exclude structural transport error, systematic boundary-condition bias, and real-world representativeness errors beyond the prescribed stochastic observation error. Results should therefore be interpreted as an information-content diagnostic, not as evidence of grid-scale resolvability or operational accuracy in real applications.

S3 OSSE design

The nature run (representing the “truth”) is generated with WRF-Chem using EDGAR CO2 emissions for March 2020. This truth simulation is performed without data assimilation and provides dynamically consistent meteorology and tracer fields used to generate synthetic observations. The OSSE is conducted for the period from 2020-03-03 to 2020-03-18, following a 48-hour spin-up (60 cycles of 6-hourly DA, starting at 00 UTC on 2020-03-03).

To define a clear and interpretable emission target for the OSSE, the corresponding inversion (prior) configuration uses EDGAR March 2020 CO2 emissions with the power-industry sector removed (“EDGAR without power-industry”). This idealized sector-removal setup provides a controlled test of how the implemented state-augmentation system responds to a known, spatially heterogeneous prior-emission perturbation (Fig. S15a). The removed EDGAR power-industry sector accounts for 36.5% of the domain-total CO2 emissions, increasing to 44.7% of emissions over South Korea, thereby imposing a substantial signal for the assimilation system to recover. Because the power-industry sector is concentrated at discrete point sources in industrial corridors rather than distributed uniformly, the experiment imposes a substantial, spatially heterogeneous signal that is, in principle, detectable. This ensures the OSSE presents a strong-signal recovery test rather than a weak-signal detection problem.

Two OSSEs are conducted under matched filter settings: (i) a 50-site network distributed primarily over South Korea and around major source regions (triangles in Fig. S15), with a minimum site spacing of approximately 30 km to increase sensitivity diversity while limiting redundancy; and (ii) a 3-site reference network corresponding to the current WMO/GAW station configuration (AMY/GSN/ULD) used in the real-data experiment. Synthetic hourly CO2 observations are sampled from the lowest model level of the nature run and assimilated within the same ±3 h temporal window used by the cycling system.

To emulate realistic measurement and representativeness uncertainty as in real applications, random observation error is introduced to each synthetic observation. These errors are assumed to be unbiased and Gaussian, specifically to match the real-data strategy described in Section 2.2: the error magnitude is dominated by the representativeness term and is parameterized as 5 % of the sampled mole fraction. Ensemble IC/BCs and emissions are randomly perturbed as described in Section 4 to introduce realistic background error relative to the unperturbed truth, while the deliberately imposed prior–truth difference is confined to the anthropogenic emission field. Furthermore, all other model inputs and ancillary datasets are kept identical to those in the real-data configuration to isolate the impact of observational density in a controlled twin setting.

S4 Assimilation configuration

This OSSE is intentionally configured as a flux-recovery diagnostic under perfect transport assumptions; meteorological variables are excluded from the augmented state to isolate the concentration–flux mapping and avoid conflating flux recoverability with transport uncertainty. The real-data experiments retain the full dual-state configuration. The assimilation step in the OSSE follows WRF-Chem/DART cycling framework described in Section 2: employing the EAKF with state augmentation to jointly estimate both GHG tracer field and surface flux parameter. The augmented state includes (i) 3-dimensional CO2 mixing ratios, and (ii) 2-dimensional anthropogenic flux scaling (or flux parameter) fields. The transport model configuration and meteorological IC/BCs follow the same setup as the nature run, with member-wise stochastic perturbations applied to the IC/BCs.

Compared to the baseline real-data configuration, a single adjustment is made for the OSSE to span the imposed truth–baseline emission gap. The only configuration parameter that differs is the prior emission uncertainty, specified as a 50 % multiplicative perturbation applied per grid cell to the EDGAR-without-power baseline. The configuration illustrates system behavior under enhanced observational coverage, using filter settings consistent with the real-data configuration (observation errors, localization, inflation) rather than idealized perfect-knowledge parameters. This design isolates the effect of expanded network density (the controlled variable) rather than conflating it with truth-tuned parameter optimization.

S5 Emission estimation under a dense surface observational network

OSSE performance is evaluated against the known truth using diagnostics that target emissions. We examine the South Korea–integrated CO2 emission budget to quantify recovery of the imposed missing-sector signal in the best-observed region. Under perfect-model assumptions, the OSSE indicates that the assimilation partially recovers the national-scale CO2 emission budget. We report both the mean flux correction integrated over the OSSE period and the equivalent annualized rate. Over South Korea, the gap between the truth (“control”, ~688 Mt yr⁻¹) and prior emissions (~384 Mt yr⁻¹) is approximately 305 Mt yr⁻¹. Averaged over the OSSE cycling period, the posterior emission closes 264 Mt yr⁻¹ of this gap (a recovery rate of 86.5%). This is also observable in the cycle-to-cycle posterior trajectory in comparison with the 3-site reference OSSE run (with WMO/GAW sites; a recovery rate of 25.5%), with posteriors rising further from baseline and the cycled prior toward the truth with the 50-site posterior reaching a markedly higher level. Densification therefore effectively scales the magnitude of the recovered correction (Fig. S16). Both experiments show similar cycle-scale variability patterns driven by shared synoptic-transport sensitivity rather than by the filter characteristics. This result provides an idealized reference for how national-scale emission recovery and uncertainty reduction may strengthen as surface observational coverage increases.

Spatial diagnostics from the OSSE indicate that, under perfect-model assumptions, the assimilation partially recovers the spatial structure of the imposed emission signal (Fig. S15). The posterior emission increment (posterior–prior) exhibits a distribution that is broadly consistent with the “true difference” associated with the excluded power-industry emissions (compare Fig. S15a with Fig. S15d).  The strongest adjustments occur in regions with the largest imposed emission differences and frequent downwind sampling by the expanded observing network. Increments remain negligible over remote cells far from the observation network (Fig. S15d), indicating that the prescribed localization effectively suppresses spurious distant corrections from sampling-noise correlations. Within the signal-bearing region, increments are predominantly positive, consistent with the imposed prior–truth direction, with no systematic wrong-direction corrections. Correspondingly, posterior emission uncertainty reductions, quantified by the decrease in ensemble spread (standard deviation) relative to the prior, are concentrated and strongest near the synthetic site clusters and major source regions, peaking above 60% over the densely observed source corridor over South Korea (Fig. S15c). The prior ensemble spread (Fig. S15b) is small but non-zero over the marginal sea cells, reflecting the imposed multiplicative emission perturbation; the analysis increments at these locations therefore lie within the prior uncertainty. Spatial maps of cycle-mean posterior emission error show that the assimilation directs its strongest emission updates toward the imposed point-source regions in both experiments (Fig. S17). In practice, localized DA transfers observational information into the emission field at the regional spatial scale set by the localization radius: with the imposed signal dominated by discrete large point sources, the innovations project as ensemble-supported regional corrections aligned with the underlying source structure rather than as cell-level adjustments at the individual point sources. Negative error magnitudes are systematically smaller in the 50-site experiment than in the 3-site reference, consistent with the higher national emissions recovery.

The controlled twin experiment provides a compact technical illustration of framework behavior under expanded observational coverage. Using a dense 50-site synthetic surface network, the system produces coherent analysis increments and spatially structured emission responses over the well-sampled region despite the large prescribed prior-truth emission gap, reflected in larger posterior uncertainty reduction. Because emission updates are formulated with a prior-proportional (multiplicative) error structure, recoverability is inherently limited where the perturbed emissions are low or near zero. The residual gap and broad posterior ensemble envelope reflect the difficulty of constraining such a hard target imposed here; such degree of prior misspecification is unlikely to be encountered in real applications.

These results were achieved using filter settings consistent with the real-data configuration, rather than truth-tuned settings. Under perfect-model assumptions, the augmented-state framework responds to increased observational information content in the expected direction. The OSSE therefore provides a controlled complement to the real-data demonstration in Section 6, rather than a proof of operational robustness.

Figure S15. Temporal-mean OSSE diagnostics for CO2 emissions using the expanded 50-site synthetic surface network over the OSSE DA cycling period from 2020-03-03 to 2020-03-18. (a) Imposed target (“true difference” in CO2 emissions), defined as EDGAR (full) minus EDGAR without the power-industry sector. (b) Prior emission uncertainty, expressed as the time-averaged prior ensemble spread (standard deviation) of CO2 emissions. (c) Posterior uncertainty reduction for CO2 emissions expressed as the percent decrease in ensemble spread (standard deviation) relative to the prior. (d) Analysis increment (posterior minus prior) in CO2 emissions from the OSSE inversion.  Triangles denote the locations of the 50 synthetic surface observation sites. All emission panels are shown in flux unit of MtCO2 km−2 yr−1. Emissions are displayed on a logarithmic color scale to visualize the highly skewed, point-source-dominated imposed signal, and panel (d) is displayed on a symmetric diverging logarithmic color scale; red (blue) shading indicates an increase (decrease) in the posterior relative to the prior.

Figure S16. Domain-total CO2 emission convergence over South Korea during the 15 days of OSSE cycling window (03-18 March 2020) for (a) the 50-site experiment and (b) the 3-site (AMY/GSN/ULD) reference experiment. Both experiments use identical filter settings (N = 20 ensemble members, 0.025 rad / ~159 km localization half-width, 50 % prior emission uncertainty); the only difference between panels is the number of synthetic observation sites. Curves show: the truth (black dashed, daily mean of the full EDGAR reference field), the initial baseline (gray dotted, EDGAR with the power-industry sector removed, defining the imposed truth–prior emission gap to be recovered), the cycled prior ensemble mean (thin solid), and the posterior ensemble mean (thick solid with circle markers); the shaded band shows the posterior ±1σ ensemble spread. Time-averaged emission-space recovery rate over the cycling window, defined as (posterior – baseline) / (truth – baseline): 86.5 % for the 50-site experiment and 25.5 % for the 3-site reference experiment.

Figure S17. Cycle-mean posterior CO2 emission error (posterior − truth, MtCO2 km⁻² yr⁻¹) averaged over the OSSE cycles (3–18 March 2020), for (a) the 50-site experiment and (b) the 3-site (AMY/GSN/ULD) reference experiment. Color scale is symmetric logarithmic; red (blue) shading indicates posterior emissions higher (lower) than truth.