# Supplemental Information for

Bomb-radiocarbon signal suggests that soil carbon contributes to chlorophyll a in archival oak leaves

# Naoto F. Ishikawa1, 2, †, Hisami Suga1, Tessa S. van der Voort2, Reto Nyffeler3, Nanako O. Ogawa1, Negar Haghipour2, 4, Lukas Wacker4, Timothy I. Eglinton2 and, Naohiko Ohkouchi1

1Japan Agency for Marine-Earth Science and Technology, Yokosuka 237-0061, Japan

2Department of Earth Sciences, ETH Zürich, 8092 Zürich, Switzerland

3Department of Systematic and Evolutionary Botany, University of Zürich, 8008 Zürich, Switzerland

4Laboratory for Ion Beam Physics, ETH Zürich, 8093 Zürich, Switzerland

†Corresponding author. E-mail: ishikawan@jamstec.go.jp

Running head: Δ14C of chlorophyll a in archival oak leaves

Supplemental Text 1. Purity Assessment

To identify and characterize impurity carbon in the purified Pheo a fractions, three additional assessments based on (i) diode array detector (DAD), (ii) Orbitrap MS, and (iii) GC/MS spectra were performed. Assessment (i) was subject to all eight samples, while assessment (ii) subject to 1952, 1968, 1973, 1982, and 1995 samples and assessment (iii) subject to 1952 and 1968 samples due to availability of leftover materials after CSRA.

(i) DAD spectrum

The DAD spectrum of all samples spanning from 300 to 720 nm absorbance was examined at the time window between 16.5 to 20.5 min when the Pheo a allomer, Pheo a, and Pheo a epimer were collected by fraction collector. The HPLC setting was the same as the second-step separation using Eclipse PAH column. Exogeneous compounds having absorbance at the 300–720 nm range were not found (Figure S3). Based on peak areas at 660 nm of Pheo a allomer, Pheo a, and Pheo a epimer, their relative proportion in each sample was estimated (Figure S4).

(ii) Orbitrap MS spectrum

Selected samples (1952, 1968, 1973, 1982, and 1995 CE) were injected to an UltiMate 3000 and Q Exactive™ Plus Hybrid Quadrupole-Orbitrap™ mass spectrometer (Thermo Fischer Scientific Inc., Waltham, MA, USA) using the electrospray ionization (ESI) method under an infusion mode. Exogeneous compounds that were ionized by ESI were not found except for Pheo a allomers (allomer 1: m/z 887.6, C55H74N4O6, allomer 2: m/z 903.6, C55H74N4O7, allomer 3: m/z 919.6, C55H74N4O8, allomer 4: m/z 935.6, C55H74N4O9) (Figure S5). The addition of oxygen to the Pheo a compound likely occurred in sample preparation and/or storage process due to an exposure to ambient air, which does not increase its C/N nor affect sample purity for CSRA measurements. Based on peak areas at exact m/z of Pheo a and its allomers, their relative proportion in each sample was estimated (Figure S6). There was a strong positive correlation in proportions of Pheo a allomers between DAD-based estimates and Orbitrap MS-based estimates (Figure S7).

(iii) GC/MS spectrum

Selected samples (1952 and 1968 CE, 2.5 µgC and 3.8 µgC, respectively) were dissolved in n-hexane, and loaded on deactivated 1 % H2O silica gel columns pre-conditioned with hexane. The N-1, N-2, and N-3 fractions were extracted with n-hexane, n-hexane/dichloromethane (50:50, v/v), and dichloromethane/methanol (90:10, v/v) respectively. N, O-bis(trimethylsilyl)trifluoroacetamide (BSTFA) was added to each vial and heated at 40 °C to derivatize the trimethylsilyl (TMS) ester. After drying with argon gas, these fractions were redissolved in 100 µL of n-hexane/dichloromethane and were injected (1 µL) to a gas chromatograph/mass spectrometer (GC/MS) (Agilent 7890A-GC with Agilent 5975C inert XL MSD) equipped with the VF-5ms column (0.25 mm ×30 m, film thickness 0.10 µm) with the electron ionization (EI) method (70eV). The oven temperature was programmed as follows: maintained at 40 °C for 2 min, raised up to 120 °C at 30 °C min-1, then to 320 °C at 6 °C min-1, and maintained at 320 °C for 20 min. Helium was used as the carrier gas with a constant flow are of 1 mL min-1.

For the N-1 and N-2 fractions, no peaks were found. For the N-3 fraction, peaks around 12–13 min appeared both in the in-house Pheo a standard and the samples, suggesting that these are ionized fractions of Pheo a compound. Exogeneous compounds that were ionized by EI were not found except for pentacyclic triterpenoid compounds (simiarenol, β-amyrin, and their derivatives) eluting around 31–32 min, which were identified by commercial standard materials (simiarenol: HY-N1308 and β-amyrin: HY-N2922, MedChemExpress, USA) (Figures S8 and S9) and by mass fragment patterns available in the literature (Elgamal et al., 1969; Galeron et al., 2016; Shiojima et al., 1992). (Figures S10–S17). The integrated peak areas of the impurities at the total ion chromatogram (TIC) were 8,992,239 and 14,639,503 for 1952 CE (Figures S10–S13) and 1968 CE (Figures S14–S17) samples, respectively.

To assess the carbon amount of simiarenol and β-amyrin (both C30H50O, 84% carbon) contained in the 1952 and 1968 Pheo a samples, their standard materials of 90 µg and 100 µg, respectively, were weighed using a microbalance (MC5, Sartorius, Germany). These were then dissolved into 2,000 µL of dichloromethane to make their final carbon concentrations of 38 ngC µL–1 and 43 ngC µL–1, respectively, and were injected (1 µL) to GC/MS. The peak areas at TIC were 104,533,222 and 142,408,002, which were then used to conversion coefficients for TIC peak area per ngC as 2,753,482 and 3,376,019 for simiarenol and β-amyrin, respectively. Since simiarenol and β-amyrin were the major impurities contained in the 1952 and 1968 Pheo a samples, respectively, the conversion coefficients were then applied to their respective TIC peak areas to derive 0.33 µgC and 0.43 µgC. These amounts divided by their respective start amounts (2.5 µgC and 3.8 µgC) yield the percentages of simiarenol and β-amyrin as 13% and 11% for the 1952 and 1968 Pheo a samples, respectively.

Finally, the percentages of simiarenol and β-amyrin based on the GC/MS analysis were compared with the impurity carbon percentages estimated by C/N and Equation (4) (13% and 16% for the 1952 and 1968 Pheo a samples, respectively). The differences between the two estimates (0.2% and 4.4% for the 1952 and 1968 Pheo a samples, respectively) were smaller than our purity criterion (< 4.5%, 1σ) based on the C/N analytical error. Therefore, there is no evidence that the impurity in the purified Pheo a samples has carbon other than simiarenol and β-amyrin. The details of comparison between triterpenoid carbon and impurity carbon are summarized in Table S1.

Supplemental Text 2. Procedural Blank Analysis

We estimated the size of excess carbon associated with the entire experimental and analytical procedure (hereafter procedural blank, PB) (Fig. 2). The carbon mass (µgC) and Δ14C (‰) of PB (MPB and Δ14CPB, respectively) have three components as follows.

MPB=MA+MB+MC,						(S1)

∆14CPB=MA×∆14CA+MB+MC×∆14CB+CMA+MB+MC,					(S2)

where MA, MB, MC, Δ14CA, and Δ14CB+C are the carbon mass (µgC) and Δ14C (‰) of (A) Wet chemistry blank, (B) EA/AMS capsule blank, and (C) EA/AMS machine blank, respectively.

MA and MB were estimated as 0.04 ± 0.08 µgC and 0.08 ± 0.01 µgC, respectively, using nano EA/IRMS. In brief, 200 µL of trichloromethane (TCM) was added to the vial that passed through (A) wet chemistry procedure using a precleaned glass syringe, 20 µL of which was transferred into a pre-cleaned (washed with DCM three times) tin capsule (3 mm diameter, 6 mm height, and 25 µL volume, P/N 84.9906.26, Lüdi Swiss, Switzerland) using a pre-cleaned glass syringe on a hot plate set at 80 ºC. Once dried, the tin capsule was introduced to the autosampler of nano EA/IRMS. Using the regression line between the nano EA/IRMS peak areas and carbon amounts of known working standards (L-tyrosine: BG-T, Tayasu et al., 2011), the (A) Wet chemistry blank MA and (B) EA/AMS capsule blank MB values were quantified for the above dried capsules and blank tin capsules, respectively (Figure S21). Four repeated measurements were performed.

The sum of MB and MC was estimated as 0.28 ± 0.08 µgC using MICADAS (Haghipour et al., 2019). Δ14CB+C was determined as –303 ± 24‰ by analyzing five blank tin capsules to achieve measurable carbon amounts using MICADAS (ETH analysis code 116866.2.1, F14C = 0.7034 ± 0.0239). The Δ14CA value is the only unknown in Equation S2, which might be derived from column bleed or organic solvents that had been potentially made from fossil-fuel products being depleted in 14C. The Δ14CChl value determined by MICADAS is represented using carbon mass (µgC) and Δ14C of Chl a, impurity, and PB as follows.

∆14CChl=MChl+MImpurity×∆14CChl,  corrected+MPB×∆14CPBMChl+MImpurity+MPB.			(S3)

Equation S3 is rewritten as follows.

∆14CChl,corrected=MChl+MImpurity+MPB×∆14CChl-MPB×∆14CPBMChl+MImpurity.		(S4)

We examined the effect of Δ14CA (i.e., wet chemistry blank) on the corrected Δ14CChl values of the four Chl a data that satisfied the purity criterion and are used for modelling (1973, 1982, 1995, and 2007 CE, Table 1) (Figure S22). Even in the most extreme case where the Δ14CA was –1,000‰, the corrected Δ14CChl values were within the MICADAS analytical error (± 8‰, 1σ) of their respective uncorrected Δ14CChl values. Therefore, we concluded that the procedural blank does not analytically influence the observed Δ14CChl values.

An aliquot (20 µL) of (A) Wet chemistry blank (dissolved in 200 µL TCM) was split into a pre-cleaned glass vial and dried. BSTFA was added to the vial and heated at 40 °C to derivatize the TMS ester. After drying with argon gas, the TMS ester was redissolved in 100 µL of dichloromethane and was injected (1 µL) to GC/MS with the same method being mentioned in the purity assessment. We found trace amounts of benzoic acid TMS derivative (Figure S23) and diethyl phthalate (Figure S24), which may have been derived from somewhere in the experimental procedure.

Supplemental Text 3. Sensitivity Analysis

Our assumption of the leaf turnover time (TQ, set as 1.5 years) refers to a previous study working on Δ14C analysis of the Quercus oak (Ichie et al., 2013). We carried out a sensitivity analysis by tweaking the TQ values from 0.5 to 5.0 years, and no substantial change was observed (Figure S25), suggesting that the model estimates are insensitive to the TQ values at this range. It should be noted that the leaf turnover time ≥ 5.0 years is unlikely because such an endmember cannot explain the 1965 and 1966 data (Figure S2).

Supplemental References

Elgamal, M. H. A., Fayez, M. B. E., & Kemp, T. R. (1969). The mass spectra of some triterpenoid dehydration products. Organic Mass Spectrometry, 2(2), 175–194. https://doi.org/10.1002/oms.1210020204

Galeron, M. A., Vaultier, F., & Rontani, J. F. (2016). Oxidation products of α- And β-amyrins: Potential tracers of abiotic degradation of vascular-plant organic matter in aquatic environments. Environmental Chemistry, 13(4), 732–744. https://doi.org/10.1071/EN15237

Haghipour, N., Ausin, B., Usman, M. O., Ishikawa, N., Wacker, L., Welte, C., Ueda, K., & Eglinton, T. I. (2019). Compound-Specific Radiocarbon Analysis by Elemental Analyzer–Accelerator Mass Spectrometry: Precision and Limitations. Analytical Chemistry, 91(3), 2042–2049. https://doi.org/10.1021/acs.analchem.8b04491

Ichie, T., Igarashi, S., Yoshida, S., Kenzo, T., Masaki, T., & Tayasu, I. (2013). Are stored carbohydrates necessary for seed production in temperate deciduous trees? Journal of Ecology, 101(2), 525–531. https://doi.org/10.1111/1365-2745.12038

Ischebeck, T., Zbierzak, A. M., Kanwischer, M., & Dörmann, P. (2006). A salvage pathway for phytol metabolism in Arabidopsis. Journal of Biological Chemistry, 281(5), 2470–2477. https://doi.org/10.1074/jbc.M509222200

Shiojima, K., Arai, Y., Masuda, K., Takase, Y., Ageta, T., & Ageta, H. (1992). Mass Spectra of Pentacyclic Triterpenoids. Chemical and Pharmaceutical Bulletin, 40(7), 1683–1690. https://doi.org/10.1248/cpb.40.1683

Tayasu, I., Hirasawa, R., Ogawa, N. O., Ohkouchi, N., & Yamada, K. (2011). New organic reference materials for carbon- and nitrogen-stable isotope ratio measurements provided by Center for Ecological Research, Kyoto University, and Institute of Biogeosciences, Japan Agency for Marine-Earth Science and Technology. Limnology, 12(3), 261–266. https://doi.org/10.1007/s10201-011-0345-5

Vavilin, D., & Vermaas, W. (2007). Continuous chlorophyll degradation accompanied by chlorophyllide and phytol reutilization for chlorophyll synthesis in Synechocystis sp. PCC 6803. Biochimica et Biophysica Acta - Bioenergetics, 1767(7), 920–929. https://doi.org/10.1016/j.bbabio.2007.03.010

| Tabe S1. Comparison between triterpenoid carbon and impurity carbon |  |  |  |
| --- | --- | --- | --- |
| Triterpenoid Standards | Simiarenol | β-amyrin | Notes |
| Formula | C30H50O | C30H50O | MW: 426.72 |
| %C | 84% | 84% | (A) |
| Weighed (µg) | 90 | 100 | (B) |
| Dichloromethane total volume (µL) | 2000 | 2000 | (C) |
| Concentration (ngC µL–1) | 38 | 42 | (D) = (A)x(B)/(C)x1000 |
| GC/MS Injection volume (µL) | 1 | 1 | (E) |
| TIC area on GC/MS | 104533222 | 142408002 | (F) |
| TIC area ngC–1 | 2753482 | 3376019 | (G) = (F)/(D)x(E) |
| Purified Pheo a Sample | 1952 CE | 1968 CE | Notes |
| µgC analyzed | 2.5 | 3.8 | (H) |
| Dichloromethane total volume (µL) | 100 | 100 | (I) |
| Concentration (ngC µL–1) | 25 | 38 | (J) = (H)/(I)x1000 |
| GC/MS Injection volume (µL) | 1 | 1 | (K) |
| TIC area on GC/MS | 8992239 | 14639503 | (L) |
| µgC impurity | 0.33 | 0.43 | (M) = (L)/(G)x(I)/(K)x1000 |
| %Triterpenoid Carbon | 13% | 11% | (N) = (H)/(M) |
| C/N expected for Pheo a | 11.8 | 11.8 | (O) = 55x12.01/4/14.0067 |
| C/N observed on EA/IRMS | 13.6 | 14.0 | (P) |
| %Impurity Carbon | 13% | 16% | (Q) = 1 – (O)/(P), See Equation (4) |
| Comparison between triterpenoids and impurities | 0.2% | 4.4% | (R) = (Q) – (N) |
| TIC: Total ion chromatogram |  |  |  |

FIGURE LEGENDS

Figure S1. Summary of key reactions in chlorophyll a anabolism and catabolism (Ischebeck et al., 2006; Vavilin & Vermaas, 2007) highlighted in the present study.

Figure S2. The difference in turnover times of the carbon source on the Δ14C vs. calendar year biplot space.

Figure S3. HPLC/DAD spectrums of Pheo a collected through separation using the C18 (1st) and PAH (2nd) columns in (a) 1952 CE, (b) 1965 CE, (c) 1966 CE, (d) 1968 CE, (e) 1973 CE, (f) 1982 CE, (g) 1995 CE, and (h) 2007 CE. Dashed lines indicate fraction collection window (16.5–20.5 min).

Figure S4. DAD peak area (660 nm) percentage of Pheo a allomer (16.5–17.8 min), Pheo a allomer epimer, Pheo a (17.8–19.2 min), and Pheo a epimer (19.2–20.5 min).

Figure S5. Orbitrap MS chromatograms of Pheo a (m/z 871.6, C55H74N4O5) in (a) an in-house standard, (b) 1952 CE, (c) 1968 CE, (d) 1973 CE, (e) 1982 CE, and (f) 1995 CE.

Figure S6. Orbitrap MS peak area percentage of Pheo a and its allomers.

Figure S7. Relationship between DAD peak area (660 nm) percentage and Orbitrap MS peak area percentage of Pheo a and its allomers. The strong positive correlation corroborates that the compounds are Pheo a derivatives (allomers).

Figure S8. (a) Total ion chromatogram and (b) mass spectrum of the simiarenol standard. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S9. (a) Total ion chromatogram and (b) mass spectrum of the β-amyrin standard. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S10. Total ion chromatogram of the N-3 fraction (a) in an in-house standard Pheo a and (b) in Pheo a extracted from the Quercus pubescens leaf collected in 1952 CE and purified for CSRA.

Figure S11. (a) Total ion chromatogram and (b) mass spectrum of the first unknown peak of the N-3 fraction in Pheo a in 1952 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S12. (a) Total ion chromatogram and (b) mass spectrum of the second unknown peak of the N-3 fraction in Pheo a in 1952 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S13. (a) Total ion chromatogram and (b) mass spectrum of the third unknown peak of the N-3 fraction in Pheo a in 1952 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S14. Total ion chromatogram of the N-3 fraction (a) in an in-house standard Pheo a and (b) in Pheo a extracted from the Quercus pubescens leaf collected in 1968 CE and purified for CSRA.

Figure S15. (a) Total ion chromatogram and (b) mass spectrum of the first unknown peak of the N-3 fraction in Pheo a in 1968 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S16. (a) Total ion chromatogram and (b) mass spectrum of the second unknown peak of the N-3 fraction in Pheo a in 1968 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S17. (a) Total ion chromatogram and (b) mass spectrum of the third unknown peak of the N-3 fraction in Pheo a in 1968 CE. The asterisk denotes the peak for which the mass spectrum is obtained.

Figure S18. The observed C/N ratios (right axis in red) and blank carbon % (left axis in blue) for Chl a purified from Quercus leaf samples plotted against collection year CE. Error bars indicate 1σ uncertainties. The dashed red line indicates expected C/N ratios of Chl a (11.8). The four older samples collected in 1952, 1965, 1966, and 1968 CE showed significantly higher C/N and blank carbon % than 11.8 and 0, respectively, while the four newer samples collected in 1973, 1982, 1995, and 2007 CE did not.

Figure S19. Heatmaps of the difference (ΔΔ14C) between observed and modelled Δ14CChl values on a biplot for soil turnover time (Ts, years) versus soil proportion (Ps, %) for each of the eight samples collected in different years. The ΔΔ14C value larger than the 2σ analytical error of CSRA (>16‰) was not considered in this plot. The white arrows denote the smallest ΔΔ14C values (i.e., the most plausible models).

Figure S20. The ΔΔ14C heatmap that overlayed all the eight heatmaps in Figure S12. The arithmetic mean of the ΔΔ14C values from the eight years are shown. The white arrow denotes the smallest ΔΔ14C value (i.e., the most plausible model).

Figure S21. The Faraday cup 2 (m/z 44, CO2) peaks for 1 µgC working standard, AMS tin capsule, and procedural blank (ten times diluted) on overlaid nano-EA/IRMS chromatograms.

Figure S22. The effect of wet chemistry blank Δ14C on the corrected Δ14CChl values of the four Chl a data that satisfied the purity criterion and are used for modelling (1973, 1982, 1995, and 2007 CE, solid horizontal lines). Dashed lines represent the corrected Δ14CChl value as a function of the wet chemistry blank Δ14C (from –1,000 to 0‰) with the MICADAS analytical error (± 8‰, 1σ). The black vertical line and shading denote the AMS capsule and machine blank Δ14C (–303 ± 24‰).

Figure S23. (a) Total ion chromatogram of the procedural blank (TMS reagent background subtracted) and (b) mass spectrum of benzoic acid TMS derivative (6.3 min).

Figure S24. (a) Total ion chromatogram of the procedural blank (TMS reagent background subtracted) and (b) mass spectrum of diethyl phthalate (10.3 min).

Figure S25. Sensitivity analysis of the two-pool mixing model for the change in its two outputs; the most plausible (a) proportion of soil carbon PS and (b) turnover time of soil carbon TS in response to the change in Quercus leaf turnover time TQ values from 0.5 to 5.0 years. Mean and standard deviation of 1973, 1982, 1995, and 2007 data are shown.

Figure S1

Figure S2

Figure S3a

Figure S3b

Figure S3c

Figure S3d

Figure S3e

Figure S3f

Figure S3g

Figure S3h

Figure S4

Figure S5a

Figure S5b

Figure S5c

Figure S5d

Figure S5e

Figure S5f

Figure S6

Figure S7

Figure S8

Figure S9

Figure S10

Figure S11

Figure S12

Figure S13

Figure S14

Figure S15

Figure S16

Figure S17

Figure S18

Figure S19

Figure S20

Figure S21

Figure S22

Figure S23

Figure S24

Figure S25