Supplement Information for:

A Modular Methane Mapping Tool (M3T): Introduction to the M3T R package

Kristian D. Hajny1,§, Joseph Pitt3, Paul B. Shepson1

1School of Marine and Atmospheric Sciences, Stony Brook University, Stony Brook, NY, 11794, USA

2Atmospheric Chemistry Research Group, School of Chemistry, University of Bristol, Bristol, UK.
§Now at GTI Energy, Des Plaines, IL, 60018, USA

Correspondence to: Kristian D. Hajny (kris.hajny@gmail.com)

SI table S1. GEPA sectors used as is in M3T.

| GEPA code | GEPA Sector | Thermogenic |
| --- | --- | --- |
| 1A | Combustion Mobile | yes |
| 1B1a | Abandoned Coal | yes |
| 1B1a | Surface Coal | yes |
| 1B1a | Underground Coal | yes |
| 1B2a | Petroleum Systems Exploration | yes |
| 1B2a | Petroleum Systems Production | yes |
| 1B2a | Petroleum Systems Refining | yes |
| 1B2a | Petroleum Systems Transport | yes |
| 1B2ab | Abandoned Oil Gas | yes |
| 1B2b | Natural Gas Exploration | yes |
| 1B2b | Natural Gas Processing | yes |
| 1B2b | Natural Gas Production | yes |
| 2B8 | Industry Petrochemical | yes |
| 2C2 | Industry Ferroalloy | yes |
| 3A | Enteric Fermentation | no |
| 3B | Manure Management | no |
| 3C | Rice Cultivation | no |
| 3F | Field Burning | no |
| 5A1 | Landfills Industrial | no |
| 5B1 | Composting | no |

# S1. Datasets

The default configuration settings used in M3T are summarized below, as well as in the package documentation.

- Throughout

  - The source for all datasets is set to M3T other than those used as is from publicly available datasets

    - GEPA (Maasakkers et al., 2023; McDuffie et al., 2023) and Vulcan v4.0 (Gurney et al., 2025a, b) are set to download

  - M3T will use Vulcan v4.0 (Gurney et al., 2025a, b) by default rather than ACES v2.0 (Gately and Hutyra, 2022, 2017)

    - There is no strong reason to justify this. Vulcan covers a wider range of years (2010 to 2022) compared to ACES (2012 to 2017). Vulcan has also existed for longer and been updated more recently.

- Municipal Solid Waste

  - The residual between the GHGI and the GHGRP national total is evenly distributed to all LMOP facilities by default. A constant emission factor for each facility can instead be set.

  - The generation-first method (HH-6) is used by default over the collection-first (HH-8) method or reported data for GHGRP facilities

    - Stark et al. (2024) and Balasus et al. (2025) discussed that the generation-first (HH-6) method shows both emissions and trends that better match some top-down studies.  However, other studies have discussed stronger correlations with the underlying data used to inform either method and this could be used to develop improved methods in the future (Howell et al., 2025; Krause et al., 2025; Srikanth et al., 2025).

- Natural Gas Distribution

  - Weller et al. (2020) emission factors are used for mains pipelines by default

  - The Fischer et al. (2018) emission factor in terms of residential gas consumption is used for residential post-meter by default

  - The commercial post-meter emissions are calculated equivalently to residential, but have an emission factor of 0 by default given the lack of data

  - GHGI emission factors are used for metering and regulating stations, maintenance events, meters, and services

  - The state-scale method is used by default as the LDC-scale method requires manual input and the domain-scale method is inappropriate for larger domains

- Natural Gas Transmission

  - GHGI emissions and activity data is used to calculate pipeline and transmission compressor emission factors by default

  - There are no method variations

- Stationary Combustion

  - Emission factors from the IPPC (Gómez et al., 2006) are used by default with the exception of natural gas for electric production which is based on Hajny et al. (2019)

  - The state-scale method is used by default as the domain-scale method is inappropriate for larger domains

- Wastewater

  - The Moore et al. (2023, 2025) log-log linear relationship between flow and emissions is the default rather than downscaling GHGI total emissions

    - Moore et al. (2023, 2025) used recent measurement campaigns with well-established and well-documented methods and demonstrated ~2x the emissions as currently estimated in the GHGI

  - GHGI non-septic emissions by year are used by default if set to use the GHGI downscaling method for municipal wastewater treatment plants

  - The DMR dataset is used as activity data for municipal wastewater treatment plants by default rather than the CWNS

    - The national total activity data is similar between the two, but there are differences at smaller spatial scales in both the activity data and facility locations. While the EPA is required to conduct the CWNS periodically in coordination with states, it is a voluntary survey that may include underreporting or nonreporting facilities/states (U.S. Code, 1972; Clean Watersheds Needs Survey (CWNS) – 2022 Report and Data, 2025). The DMR by comparison, are legally required (Electronic Code of Federal Regulations, 2015). The feedback below is from the ECHO help team regarding the differences between the CWNS and DMR (ECHO Support Team at ERG (EPA Contractor)).

    - “The Clean Water Needs Survey (CWNS) data and ECHO values may differ as these data are reported differently. The DMR Pollutant Loading Report on ECHO pulls water data from ICIS-NPDES, which contains discharge monitoring report data submitted by NPDES permitted facilities. The three flow values seen on this report are reported at either the time of permit application or is calculated using flow data reported in ICIS-NPDES. The Average Facility Flow (DMR) value is the average flow from all external discharge points (outfalls) at the facility during the selected reporting year, calculated in the Loading Tool from flow data reported on DMRs. For more information about the data, please see the DMR Pollutant Loading Report Help (https://echo.epa.gov/help/loading-tool/reports/pollutant-loading-report-help-dmr)... The CWNS data is based on a comprehensive survey conducted by EPA every four years to get a comprehensive assessment of the capital costs (or needs) to meet the water quality goals of the CWA and address what quality and water quality related public health concerns. We recommend reviewing the CWNS 2012 Data Dictionary (https://www.epa.gov/sites/default/files/2016-01/documents/cwns-2012-data_dictionary2.pdf)...”

  - GHGI septic emissions by year are used by default for the national septic method

  - The national septic method is used by default rather than the state level method

    - There is no strong reason to justify this choice. It was selected as it relies on fewer datasets, just the NLCD and GHGI. The state method relies on census population and septic fraction data, the NLCD, and a GHGI per capita septic EF.

  - The GHGI septic per capita EF is used by default if set to use the state septic method

    - U.S. Census national and state septic fractions are used by default for the state septic method

  - The national total “Developed, Open” and “Developed, Low Intensity” is calculated directly using NLCD data for CONUS and a linear regression using the limited AK NLCD data available by default

- Wetlands and Inland Waters

  - Rosentreter et al. (2021) emission factors are used by default for freshwater emissions

  - SOCCR1 and 2 EFs are used by default for the SOCCR methods, but other EFs could be used by changing these EFs and using the SOCCR methods. SOCCR1 does not vary by watershed while SOCCR2 does.

  - WetCHARTs is used by default

    - There is no strong reason to justify this choice. WetCHARTs is composed of numerous model runs and is, to our knowledge, more commonly used to represent wetland emissions in the literature

- Combine Sectors

  - The default is to only produce the min, mean, and max value across the variations chosen. Separating into thermogenic and non-thermogenic totals and calculating all possible combinations across variations will not be relevant for all users and will considerably increase processing time.

# S2. Aligning GHGRP and HIFLD Transmission Compressor Datasets

The GHGRP and HIFLD do not have common identification codes. To define matching facilities, the nearest HIFLD facility was calculated for each GHGRP facility, including all years of GHGRP data. All years were considered as names and locations can change over time, and some facilities may not report every year as emissions may be low enough some years and not others. This was taken as an initial match and was then checked by comparing facility names manually. Though exact name matches were uncommon, reasonable matches (e.g., MGT Potomac 2118 and #2118 Potomac or EPNG Belen Compressor Station and Belen) were common. Facilities that were within 1 km were considered a match even if there was no clear name match. Across facilities with a match, 481 (82%) GHGRP facilities had a HIFLD facility within 1 km, 75 (13%) had a HIFLD facility between 1 and 10 km away, and 29 (5%) had a HIFLD facility >10 km away, up to a maximum of 55.5 km. For context, GHGRP facility locations can also be updated over time. Across all facilities that had at least 2 years of reported data 728 (93%) had updated locations within 1 km, 37 (5%) had updated locations between 1 and 10 km away, and 20 (3%) had updated locations > 10 km away, up to a maximum of 289 km for GHGRP ID 1002979.

# S3. Per Area and Per Capita Analysis

We investigated the ten urban areas with the largest emissions per capita and per area in addition to those with the largest emissions. These are different sets of urban areas than those with the largest emissions, though many of those with significant per capita emissions also have significant per area emissions. Those with the largest total emissions are all large, dense urban environments with areas of at least 3,000 km2 and populations of at least 3.5 million while those with the largest per capita or per area emissions are all small urban areas under 160 km2 and populations under 65 thousand, many under 10 thousand. Figures S1 and S2 show that these urban areas with large per capita or per area emissions are also each dominated by one type of point source such as landfills, natural gas transmission compressors, or wastewater treatment plants unlike the urban areas with large emissions discussed in the main text that each had a range of significant sources.

Figure S1: Bar plot equivalent to main text Figure 8 for the ten urban areas with the largest emissions per area. Bars on the left for each sector represent the average M3T output across all variations with error bars representing the range across variations. Bars on the right represent GEPA emissions.

Figure S2: Bar plot equivalent to main text Figure 8 for the ten urban areas with the largest emissions per capita. Bars on the left for each sector represent the average M3T output across all variations with error bars representing the range across variations. Bars on the right represent GEPA emissions.

Figure S3: CONUS maps with red diamonds showing the urban areas with the largest emissions or largest emissions per population or area as per their titles. Data for all 30 urban areas are provided in Tables S1 - S3.

SI table S2. Emissions data for the ten urban areas with the largest emissions

| Urban Area | State(s) | Mean Emissions (Gg/year) | Area (km2) | Emissions Per Area (Mg/km2/year) | Population | Emissions Per Capita (kg/person/year) |
| --- | --- | --- | --- | --- | --- | --- |
| New York–Newark | NY--NJ–CT | 226.699 | 8,936 | 25 | 18,351,295 | 12 |
| Chicago | IL–IN | 149.111 | 6,327 | 24 | 8,608,208 | 17 |
| Los Angeles--Long Beach--Anaheim | CA | 146.765 | 4,496 | 33 | 12,150,996 | 12 |
| Dallas--Fort Worth--Arlington | TX | 108.193 | 4,608 | 23 | 5,121,892 | 21 |
| Houston | TX | 94.634 | 4,299 | 22 | 4,944,332 | 19 |
| Philadelphia | PA–NJ–DE–MD | 82.884 | 5,132 | 16 | 5,441,567 | 15 |
| Detroit | MI | 75.766 | 3,463 | 22 | 3,734,090 | 20 |
| Atlanta | GA | 75.495 | 6,851 | 11 | 4,515,419 | 17 |
| Miami | FL | 67.012 | 3,208 | 21 | 5,502,379 | 12 |
| Boston | MA–NH–RI | 61.494 | 4,852 | 13 | 4,181,019 | 15 |

SI table S3. Emissions data for the ten urban areas with the largest emissions per area. Those in bold are urban areas that are also in SI Table S4.

| Urban Area | State(s) | Mean Emissions (Gg/year) | Area (km2) | Emissions Per Area (Mg/km2/year) | Population | Emissions Per Capita (kg/person/year) |
| --- | --- | --- | --- | --- | --- | --- |
| Iowa Park | TX | 3.632 | 7 | 485 | 6,285 | 578 |
| Kimberling City | MO | 5.152 | 12 | 441 | 3,172 | 1,624 |
| Coopersville | MI | 1.877 | 5 | 370 | 3,951 | 475 |
| Versailles | KY | 8.349 | 28 | 301 | 15,907 | 525 |
| Gonzales | CA | 0.880 | 4 | 216 | 8,174 | 108 |
| Leland | MS | 0.927 | 5 | 190 | 4,539 | 204 |
| Jersey Shore | PA | 2.858 | 18 | 158 | 9,606 | 298 |
| Battlement Mesa | CO | 0.989 | 8 | 130 | 5,071 | 195 |
| Morris | IL | 3.177 | 26 | 122 | 15,668 | 203 |
| Othello | WA | 2.450 | 20 | 120 | 11,207 | 219 |

SI table S4. Emissions data for the ten urban areas with the largest emissions per capita. Those in bold are urban areas that are also in SI Table S3.

| Urban Area | State(s) | Mean Emissions (Gg/year) | Area (km2) | Emissions Per Area (Mg/km2/year) | Population | Emissions Per Capita (kg/person/year) |
| --- | --- | --- | --- | --- | --- | --- |
| Kimberling City | MO | 5.152 | 12 | 441 | 3,172 | 1,624 |
| Iowa Park | TX | 3.632 | 7 | 485 | 6,285 | 578 |
| Versailles | KY | 8.349 | 28 | 301 | 15,907 | 525 |
| Coopersville | MI | 1.877 | 5 | 370 | 3,951 | 475 |
| Jersey Shore | PA | 2.858 | 18 | 158 | 9,606 | 298 |
| New Boston | TX | 1.367 | 12 | 117 | 4,707 | 290 |
| Beckley | WV | 15.948 | 158 | 101 | 64,022 | 249 |
| Rocky Mount | VA | 1.361 | 19 | 72 | 6,048 | 225 |
| Wisconsin Rapids | WI | 6.396 | 56 | 115 | 29,169 | 219 |
| Othello | WA | 2.450 | 20 | 120 | 11,207 | 219 |

# References

Balasus, N., Jacob, D. J., Maxemin, G., Jenks, C., Nesser, H., Maasakkers, J. D., Cusworth, D. H., Scarpelli, T. R., Varon, D. J., and Wang, X.: Satellite monitoring of annual US landfill methane emissions and trends, Environ. Res. Lett., 20, 024007, https://doi.org/10.1088/1748-9326/ada2b1, 2025.

Electronic Code of Federal Regulations: 40 Code of Federal Regulations §127, 2015.

Fischer, M. L., Chan, W. R., Delp, W., Jeong, S., Rapp, V., and Zhu, Z.: An Estimate of Natural Gas Methane Emissions from California Homes, Environ. Sci. Technol., 52, 10205–10213, https://doi.org/10.1021/acs.est.8b03217, 2018.

Gately, C. and Hutyra, L. R.: Anthropogenic Carbon Emission System, 2012-2017, Version 2, ORNL DAAC, https://doi.org/10.3334/ORNLDAAC/1943, 2022.

Gately, C. K. and Hutyra, L. R.: Large Uncertainties in Urban-Scale Carbon Emissions, Journal of Geophysical Research: Atmospheres, 122, 11,242-11,260, https://doi.org/10.1002/2017JD027359, 2017.

Gómez, D. R., Watterson, J. D., Americano, B. B., Ha, C., Marland, G., Matsika, E., Nenge Namayanga, L., Osman-Elasha, B., Kalenga Saka, J. D., and Treanton, K.: 2006 IPCC Guidelines for National Greenhouse Gas Inventories - Stationary Combustion, 2006.

Gurney, K., Dass, P., Kato, A., Gawuc, L., Aslam, B., and Sun, H.: Vulcan version 4.0 high-resolution annual fossil fuel and cement production carbon dioxide emissions in the United States for the 2010-2022 time period (4.0), https://doi.org/10.5281/zenodo.15446748, 2025a.

Gurney, K. R., Dass, P., Kato, A., Gawuc, L., Aslam, B., and Sun, H.: Vulcan version 4.0 high-resolution annual carbon dioxide emissions in the U.S. for the 2010–2022 time period, Sci Data, 12, 1946, https://doi.org/10.1038/s41597-025-06391-w, 2025b.

Hajny, K. D., Salmon, O. E., Rudek, J., Lyon, D. R., Stuff, A. A., Stirm, B. H., Kaeser, R., Floerchinger, C. R., Conley, S., Smith, M. L., and Shepson, P. B.: Observations of Methane Emissions from Natural Gas-Fired Power Plants, Environ. Sci. Technol., 53, 8976–8984, https://doi.org/10.1021/acs.est.9b01875, 2019.

Howell, K., Scarpelli, T., Duren, R., Thorpe, A., Asner, G., Heckler, J., Green, R., Brantley, H., and Cusworth, D.: Informing landfill emission reporting programs through remote sensing, https://eartharxiv.org/repository/view/10478/, 11 October 2025.

Krause, M. J., Thoma, E. D., Bryant, A., Brantley, H., MacDonald, M., Green, R., and Thorneloe, S.: A High-Resolution Satellite Survey of Methane Emissions from 60 North American Municipal Solid Waste Landfills, Environ. Sci. Technol., 59, 15080–15091, https://doi.org/10.1021/acs.est.5c01409, 2025.

Maasakkers, J. D., McDuffie, E. E., Sulprizio, M. P., Chen, C., Schultz, M., Brunelle, L., Thrush, R., Steller, J., Sherry, C., Jacob, D. J., Jeong, S., Irving, B., and Weitz, M.: A Gridded Inventory of Annual 2012–2018 U.S. Anthropogenic Methane Emissions, Environ. Sci. Technol., 57, 16276–16288, https://doi.org/10.1021/acs.est.3c05138, 2023.

McDuffie, E., E., Maasakkers, J., D., Sulprizio, M., P., Chen, C., Schultz, M., Brunelle, L., Thrush, R., Steller, J., Sherry, C., Jacob, D., J., Jeong, S., Irving, B., and Weitz, M.: Gridded EPA U.S. Anthropogenic Methane Greenhouse Gas Inventory (gridded GHGI), 2023.

Moore, D. P., Li, N. P., Wendt, L. P., Castañeda, S. R., Falinski, M. M., Zhu, J.-J., Song, C., Ren, Z. J., and Zondlo, M. A.: Underestimation of Sector-Wide Methane Emissions from United States Wastewater Treatment, Environ. Sci. Technol., 57, 4082–4090, https://doi.org/10.1021/acs.est.2c05373, 2023.

Moore, D. P., Li, N. P., Song, C., Zhu, J.-J., Yi, H., Tao, L., McSpiritt, J., Sevostianov, V. I., Wendt, L. P., Rojas-Robles, N. E., Hopkins, F. M., Ren, Z. J., and Zondlo, M. A.: Comprehensive assessment of the contribution of wastewater treatment to urban greenhouse gas and ammonia emissions, Nat Water, 3, 1114–1124, https://doi.org/10.1038/s44221-025-00490-z, 2025.

Rosentreter, J. A., Borges, A. V., Deemer, B. R., Holgerson, M. A., Liu, S., Song, C., Melack, J., Raymond, P. A., Duarte, C. M., Allen, G. H., Olefeldt, D., Poulter, B., Battin, T. I., and Eyre, B. D.: Half of global methane emissions come from highly variable aquatic ecosystem sources, Nat. Geosci., 14, 225–230, https://doi.org/10.1038/s41561-021-00715-2, 2021.

Srikanth, A., Stokes, S., Yang, S., and Ravikumar, A.: Understanding Trends in Methane Emissions from Landfills through Repeat Aerial Surveys, https://doi.org/10.26434/chemrxiv-2025-77s45, 21 November 2025.

Stark, B. M., Tian, K., and Krause, M. J.: Investigation of U.S. landfill GHG reporting program methane emission models, Waste Management, 186, 86–93, https://doi.org/10.1016/j.wasman.2024.05.037, 2024.

U.S. Code: 33 USC 1375: Reports to Congress; detailed estimates and comprehensive study on costs; State estimates, 1972.

Clean Watersheds Needs Survey (CWNS) – 2022 Report and Data: https://www.epa.gov/cwns/clean-watersheds-needs-survey-cwns-2022-report-and-data, last access: 21 November 2025.

Weller, Z. D., Hamburg, S. P., and von Fischer, J. C.: A National Estimate of Methane Leakage from Pipeline Mains in Natural Gas Local Distribution Systems, Environ. Sci. Technol., 54, 8958–8967, https://doi.org/10.1021/acs.est.0c00437, 2020.